###### ZTP

Objective:

Objective is to install SAOS 10.x with ZTP process. Network based ZTP process is useful when we want to ensure the same version of software is automatically installed on multiple nodes in the network. ZTP only allowed over VLAN 127, and untagged is not supported.

Procedure:

- Ensure you have DHCP environment setup is been done with either of

  - Option 66/67 to provide tftp server and filename of Command File

  - Option 125 to provide URL of Command Fil

- Ensure that the image you want to download is located on an HTTP/TFTP server

- Verify that the command file is located at the TFTP server and the config file is supplied

- Start the node if it is the first time it is being powered up

- Once the device boots up, Login to UI container using the management IP you checked in the previous step (default login/passwd displayed below):

ssh diag@x.x.x.x

username: diag password: ciena123

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

###### Verify Manual network-based installation

Objective:

Objective is to install SAOS 10.x on a system, using the manual network-based installation.

Procedure:

- Login to system console port.

- When ONIE comes up it will try to find the resource for ZTP. If you want to specify the location of the installer file manually, which is different from what it might fetch from network installer, you should stop it:

ONIE:/ # onie-discovery-stop

- Then proceed with the manual installation from the installer file from http, tftp or ftp server.

ONIE:/ # onie-nos-install http://x.x.x.x/installer.bin ONIE:/ # onie-nos-install tftp://x.x.x.x/installer.bin ONIE:/ # onie-nos-install ftp://x.x.x.x/installer.bin

Replace x.x.x.x with IP address or the URL of where the image is residing. Here is an example:

ONIE:/ # onie-nos-install http://x.x.x.x/valimar- snapshot/17-12-00-0141/meta-onie-installer-dnx/meta_01-00- 00-0181-core.bin

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

###### Verify USB-based installation of SAOS 10.x

Objective:

Objective is to install SAOS 10.x on a system using USB-based method.

Procedure:

- Ensure that the format of your USB stick is FAT32 formatted.

- Ensure that the image you want is on the USB stick

- Ensure that the image is named “onie-installer”. This is important. Currently, the load name cannot be named anything else.

- Start the node if it is the first time it is being powered up, or follow the following steps otherwise

- Login to UI container using the management IP you checked in the previous step:

ssh diag@x.x.x.x

username: diag password: ciena123

When ONIE comes up it will try to find the resource for ZTP. Wait for the installation to complete from here.

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

###### Verify System Power Cycle will maintain the configuration

Objective:

To verify that SAOS 10.x on sytem maintains configuration and operation following a disruption to power.

Procedure:

- Login to UI container using the management IP you checked in the previous step:

ssh diag@x.x.x.x

username: diag password: ciena123

- Then issue the reboot command below or power cycle the node.

  - system restart

- Once powered up verify that device can be reachable by management network, and login is re-enabled.

- Login to the box and confirm configuration is still in place.

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

###### SNMP v3 Trap Configuration

Objective:

The objective of this section is to configure SNMP v3 Trap on the SAOS cli.

SNMPv3 Trap:

      - Same as the SNMP GET parameters but additional fields are added to specify the Trap target IP.

Procedure:

snmp usm local user cienaNoAuthNoPrivUser exit

exit exit exit

snmp usm local user cienaAuthNoPrivUser auth md5 key 63:69:65:6e:61:41:75:74:68:4b:65:79 snmp usm local user cienaAuthPrivUser auth md5 key 63:69:65:6e:61:41:75:74:68:4b:65:79 snmp usm local user cienaAuthPrivUser priv des key 63:69:65:6e:61:50:72:69:76:4b:65:79

snmp vacm group cienaV3Group access "" usm no-auth-no-priv context-match exact notify-view cienaAll

snmp vacm group cienaV3Group access "" usm auth-no-priv context-match exact notify-view cienaAll

snmp vacm group cienaV3Group access "" usm auth-priv context-match exact notify-view cienaAll snmp vacm group cienaV3Group member cienaNoAuthNoPrivUser security-model usm

snmp vacm group cienaV3Group member cienaAuthNoPrivUser security-model usm snmp vacm group cienaV3Group member cienaAuthPrivUser security-model usm snmp target TestTarget2 target-params TestUsmNoAuthNoPriv udp ip 10.176.137.38 snmp target TestTarget2 tag TestTag

snmp target TestTarget3 target-params TestUsmAuthNoPriv udp ip 10.176.137.38 snmp target TestTarget3 tag TestTag

snmp target TestTarget4 target-params TestUsmAuthPriv udp ip 10.176.137.38 snmp target TestTarget4 tag TestTag

snmp target-params TestUsmNoAuthNoPriv usm security-level no-auth-no-priv user-name cienaNoAuthNoPrivUser

snmp target-params TestUsmAuthNoPriv usm security-level auth-no-priv user-name cienaAuthNoPrivUser

snmp target-params TestUsmAuthPriv usm security-level auth-priv user-name cienaAuthPrivUser snmp vacm view cienaAll include internet

snmp notify Test-NOTIFY tag TestTag type trap

Output:

- show snmp

+

SNMP NOTIFY	+

| Notification Name | Notification Tag | Notification Type |

+	+	+	+

| Test-NOTIFY	| TestTag	| trap	|

+	+	+

+

+

SNMP USM	+

| User Name	| Type | Auth Protocol | Priv Protocol | Engine ID	|

+	+	+	+	+	+

| cienaNoAuthNoPrivUser | local |	|	| 80:00:05:23:01:0A:78:67:F8:00:00 |

| cienaAuthNoPrivUser	| local | md5	|	| 80:00:05:23:01:0A:78:67:F8:00:00 |

| cienaAuthPrivUser	| local | md5	| des	| 80:00:05:23:01:0A:78:67:F8:00:00 |

+	+	+

+

+	+	+

SNMP TARGET-PARAMS	+

| Target Param Name	| Security Name | User Name	| Security Model | Security Level |

+	+	+	+	+	+

| TestUsmNoAuthNoPriv |	| cienaNoAuthNoPrivUser | usm	| no-auth-no-priv |

| TestUsmAuthNoPriv	|	| cienaAuthNoPrivUser	| usm	| auth-no-priv	|

| TestUsmAuthPriv	|	| cienaAuthPrivUser	| usm	| auth-priv	|

+	+	+

+

+	+	+

SNMP TARGET	+

| Target Name | IP Address	| Param Name	| Tags	| UDP Port | Retry Count | Timeout | Prefix Length |

+	+	+	+	+	+	+	+	+

| TestTarget2 | 10.176.137.38 | TestUsmNoAuthNoPriv | TestTag | 162	| 3	| 1500	| 32	|

| TestTarget3 | 10.176.137.38 | TestUsmAuthNoPriv	| TestTag | 162	| 3	| 1500	| 32	|

| TestTarget4 | 10.176.137.38 | TestUsmAuthPriv	| TestTag | 162	| 3	| 1500	| 32	|

+	+	+

+	SNMP VACM VIEW	+

+	+	+	+	+	+

| Viewtree Name | Subtree | Type	|

+	+	+	+

| cienaAll	| internet | include |

+	+	+	+

+	SNMP VACM GROUP MEMBER	+

| Group Name	| Security Model | Security Name	|

+	+	+	+

| cienaV3Group | usm	| cienaNoAuthNoPrivUser |

| cienaV3Group | usm	| cienaAuthNoPrivUser	|

| cienaV3Group | usm	| cienaAuthPrivUser	|

+	+	+

+

+

SNMP VACM GROUP ACCESS	+

| Group Name	| Context | Context Match | Security Model | Security Level | Read View | Notify View |

+	+	+	+	+	+	+	+

| cienaV3Group |	| exact	| usm	| no-auth-no-priv |	| cienaAll	|

| cienaV3Group |	| exact	| usm	| auth-no-priv	|	| cienaAll	|

| cienaV3Group |	| exact	| usm	| auth-priv	|	| cienaAll	|

+	+	+	+	+	+	+	+

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

###### SNMP v3 Trap Configuration

Objective:

The objective of this section is to configure SNMP v3 Trap on the SAOS cli.

SNMPv3 Trap:

      - Same as the SNMP GET parameters but additional fields are added to specify the Trap target IP.

Procedure:

snmp usm local user cienaNoAuthNoPrivUser exit

exit exit exit

snmp usm local user cienaAuthNoPrivUser auth md5 key 63:69:65:6e:61:41:75:74:68:4b:65:79 snmp usm local user cienaAuthPrivUser auth md5 key 63:69:65:6e:61:41:75:74:68:4b:65:79 snmp usm local user cienaAuthPrivUser priv des key 63:69:65:6e:61:50:72:69:76:4b:65:79

snmp vacm group cienaV3Group access "" usm no-auth-no-priv context-match exact notify-view cienaAll

snmp vacm group cienaV3Group access "" usm auth-no-priv context-match exact notify-view cienaAll

snmp vacm group cienaV3Group access "" usm auth-priv context-match exact notify-view cienaAll snmp vacm group cienaV3Group member cienaNoAuthNoPrivUser security-model usm

snmp vacm group cienaV3Group member cienaAuthNoPrivUser security-model usm snmp vacm group cienaV3Group member cienaAuthPrivUser security-model usm snmp target TestTarget2 target-params TestUsmNoAuthNoPriv udp ip 10.176.137.38 snmp target TestTarget2 tag TestTag

snmp target TestTarget3 target-params TestUsmAuthNoPriv udp ip 10.176.137.38 snmp target TestTarget3 tag TestTag

snmp target TestTarget4 target-params TestUsmAuthPriv udp ip 10.176.137.38 snmp target TestTarget4 tag TestTag

snmp target-params TestUsmNoAuthNoPriv usm security-level no-auth-no-priv user-name cienaNoAuthNoPrivUser

snmp target-params TestUsmAuthNoPriv usm security-level auth-no-priv user-name cienaAuthNoPrivUser

snmp target-params TestUsmAuthPriv usm security-level auth-priv user-name cienaAuthPrivUser snmp vacm view cienaAll include internet

snmp notify Test-NOTIFY tag TestTag type trap

Output:

- show snmp

+

SNMP NOTIFY	+

| Notification Name | Notification Tag | Notification Type |

+	+	+	+

| Test-NOTIFY	| TestTag	| trap	|

+	+	+

+

+

SNMP USM	+

| User Name	| Type | Auth Protocol | Priv Protocol | Engine ID	|

+	+	+	+	+	+

| cienaNoAuthNoPrivUser | local |	|	| 80:00:05:23:01:0A:78:67:F8:00:00 |

| cienaAuthNoPrivUser	| local | md5	|	| 80:00:05:23:01:0A:78:67:F8:00:00 |

| cienaAuthPrivUser	| local | md5	| des	| 80:00:05:23:01:0A:78:67:F8:00:00 |

+	+	+

+

+	+	+

SNMP TARGET-PARAMS	+

| Target Param Name	| Security Name | User Name	| Security Model | Security Level |

+	+	+	+	+	+

| TestUsmNoAuthNoPriv |	| cienaNoAuthNoPrivUser | usm	| no-auth-no-priv |

| TestUsmAuthNoPriv	|	| cienaAuthNoPrivUser	| usm	| auth-no-priv	|

| TestUsmAuthPriv	|	| cienaAuthPrivUser	| usm	| auth-priv	|

+	+	+

+

+	+	+

SNMP TARGET	+

| Target Name | IP Address	| Param Name	| Tags	| UDP Port | Retry Count | Timeout | Prefix Length |

+	+	+	+	+	+	+	+	+

| TestTarget2 | 10.176.137.38 | TestUsmNoAuthNoPriv | TestTag | 162	| 3	| 1500	| 32	|

| TestTarget3 | 10.176.137.38 | TestUsmAuthNoPriv	| TestTag | 162	| 3	| 1500	| 32	|

| TestTarget4 | 10.176.137.38 | TestUsmAuthPriv	| TestTag | 162	| 3	| 1500	| 32	|

+	+	+

+	SNMP VACM VIEW	+

+	+	+	+	+	+

| Viewtree Name | Subtree | Type	|

+	+	+	+

| cienaAll	| internet | include |

+	+	+	+

+	SNMP VACM GROUP MEMBER	+

| Group Name	| Security Model | Security Name	|

+	+	+	+

| cienaV3Group | usm	| cienaNoAuthNoPrivUser |

| cienaV3Group | usm	| cienaAuthNoPrivUser	|

| cienaV3Group | usm	| cienaAuthPrivUser	|

+	+	+

+

+

SNMP VACM GROUP ACCESS	+

| Group Name	| Context | Context Match | Security Model | Security Level | Read View | Notify View |

+	+	+	+	+	+	+	+

| cienaV3Group |	| exact	| usm	| no-auth-no-priv |	| cienaAll	|

| cienaV3Group |	| exact	| usm	| auth-no-priv	|	| cienaAll	|

| cienaV3Group |	| exact	| usm	| auth-priv	|	| cienaAll	|

+	+	+	+	+	+	+	+

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

###### Create New User Account

Objective:

Setup a new user account to access the system.

Procedure:

- SAOS 10.1 supports two local user groups:

  - SYSTEM_ROLE_DIAG : The user has access to CLI, NETCONF, and also to Linux shell.

  - SYSTEM_ROLE_USER : The user has access to CLI and NETCONF.

- Enter the following command on CLI to create a new local user User1:

config

system aaa authentication users user User1 config username User1 role SYSTEM_ROLE_USER password Password1

- Verify that the user has been created:

5162_001> show aaa users

+	USER ACCOUNT TABLE	+

| Username | Role	| Sessions | Lockout |

+	+	+	+	+

| diag	| SYSTEM_ROLE_DIAG |	|	|

| user	| SYSTEM_ROLE_USER |	|	|

| User1	| SYSTEM_ROLE_USER |	|	|

+	+	+	+	+

- Ensure that the user has been added to NACM group -otherwise provisioning by local user will not be allowed.

  - config

  - nacm groups group super user-name User1

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

###### DHCPv6 Client Setup

Objective:

Ensure that with DHCPv6 client enabled, the interface will acquire an IPv6 address automatically. Only stateful DHCPv6 (no SLAAC) is currently supported.

Procedure:

- Dual IPv4 and IPv6 stack on mgmt. interfaces are supported as of 10.3 and onwards. Enable DHCPv6 client on mgmtbr0. Ensure that :

  - config

  - dhcpv6-client client mgmtbr0 admin-enable true

- Configure IPv4 static IP on the mgmtbr0 as well.

  - dhcp-client client mgmtbr0 admin-enable false

  - oc-if:interfaces interface mgmtbr0 ipv4 address address x.x.x.x config ip x.x.x.x prefix-length 20

  - rib vrf default ipv4 x.x.x.x/8 next-hop x.x.x.x description "lab default"

Ex:

dhcp-client client mgmtbr0 admin-enable false

oc-if:interfaces interface mgmtbr0 ipv4 address address 10.181.36.85 config ip 10.181.36.85 prefix-length 20

rib vrf default ipv4 10.0.0.0/8 next-hop 10.181.32.1 description "lab default"

- Display the IPv6 and IPv4 interfaces on the node.

  - 5170-0037> show ipv6 interfaces brief

  - 5170-0037> show ip interfaces brief

5170-0037> show dhcpv6

+-------- DHCPV6 CLIENT CONFIGURATION	+

| Name	| Value	|

+	+	+

| Interface Name	| mgmtbr0 |

| Admin State	| Enabled |

| Rapid Commit	| Enabled |

| Requested Preferred Lifetime (s) | 0	|

| Requested Valid Lifetime (s)	| 0	|

| Option	|	|

|	DNS Server List	| Enabled |

|	Domain Search List	| Enabled |

|	Posix Time Zone	| Disabled |

|	TZDB Time Zone	| Enabled |

|	NTP Server	| Enabled |

|	Bootfile URL	| Enabled |

+	+	+

| Interface Name	| remote	|

| Admin State	| Enabled |

| Rapid Commit	| Enabled |

| Requested Preferred Lifetime (s) | 0	|

| Requested Valid Lifetime (s)	| 0	|

| Option	|	|

|	DNS Server List	| Enabled |

|	Domain Search List	| Enabled |

|	Posix Time Zone	| Disabled |

|	TZDB Time Zone	| Enabled |

|	NTP Server	| Enabled |

|	Bootfile URL	| Enabled |

+	+	+

+	DHCPV6 CLIENT STATE	+

| Name	| Value	|

+	+	+

| Interface Name	| remote	|

| Oper State	| Enabled	|

| DHCPv6 State	| preinit	|

| Config State	| stateful	|

| Renewal (T1) Time (s)	|	|

| Renewal (T1) Time Remaining (s)	|	|

| Rebinding (T2) Time (s)	|	|

| Rebinding (T2) Time Remaining (s) |	|

| ¦ ¦ ¦ ¦ | Preferred Lifetime (s) Preferred Lifetime Remaining Valid Lifetime (s) Valid Lifetime Remaining (s) | (s) | ¦ ¦ ¦ ¦ |  | ¦ ¦ ¦ ¦ |
| --- | --- | --- | --- | --- | --- |
| ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ + ¦ | DHCPv6 Server DUID Option Value DNS Server List Domain Search List Posix Time Zone TZDB Time Zone NTP servers Boot File URL Interface Name |  | ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ + ¦ | mgmtbr0 | ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ + ¦ |
| ¦ ¦ ¦ ¦ | Oper State DHCPv6 State Config State Renewal (T1) Time (s) |  | ¦ ¦ ¦ ¦ | Enabled bound stateful 302400 | ¦ ¦ ¦ ¦ |
| ¦ | Renewal (T1) Time Remaining | (s) | ¦ | 241733 | ¦ |
| ¦ | Rebinding (T2) Time (s) |  | ¦ | 453600 | ¦ |
| ¦ | Rebinding (T2) Time Remaining (s) |  | ¦ | 392933 | ¦ |
| ¦ | Preferred Lifetime (s) |  | ¦ | 604800 | ¦ |
| ¦ | Preferred Lifetime Remaining (s) |  | ¦ | 544133 | ¦ |
| ¦ | Valid Lifetime (s) |  | ¦ | 604800 | ¦ |
| ¦ | Valid Lifetime Remaining (s) |  | ¦ | 544133 | ¦ |
| ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ + | DHCPv6 Server DUID Option Value DNS Server List Domain Search List Posix Time Zone TZDB Time Zone NTP servers Boot File URL |  | ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ + | 0:1:0:1:25:22:d:24:0:0:5e:0:1:81 | ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ + |

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

    - SNMP v2c Trap Configuration

Objective:

The objective of this section is to configure SNMP v2c Trap on the SAOS cli.

      - Same as the SNMP GET parameters but additional fields are added to specify the Trap target IP.

Procedure:

snmp community t1 security-name cienaSecurityV2c text-name cienaV2cCommunity snmp vacm view cienaAll include internet

snmp vacm group cienaGroup access "" v2c no-auth-no-priv context-match exact notify-view cienaAll

snmp vacm group cienaGroup member cienaSecurityV2c security-model v2c snmp target TestTarget1 target-params TestParamsV2c udp ip 10.176.137.38 snmp target-params TestParamsV2c v2c security-name cienaSecurityV2c snmp target TestTarget1 tag TestTag

snmp notify Test-NOTIFY type trap tag TestTag

Output:

- show snmp

+

SNMP NOTIFY	+

| Notification Name | Notification Tag | Notification Type |

+	+	+	+

| Test-NOTIFY	| TestTag	| trap	|

+	+	+	+

+	SNMP COMMUNITY	+

| Community Index | Community Name	| Security Name	| Transport Tag |

+	+	+	+	+

| t1	| cienaV2cCommunity | cienaSecurityV2c |	|

+	+	+	+	+

+	SNMP TARGET-PARAMS	+

| Target Param Name | Security Name	| User Name | Security Model | Security Level |

+	+	+	+	+	+

| TestParamsV2c	| cienaSecurityV2c |	| v2c	| no-auth-no-priv |

+	+	+

+

+	+	+

SNMP TARGET	+

| Target Name | IP Address	| Param Name	| Tags	| UDP Port | Retry Count | Timeout | Prefix Length |

+	+	+	+	+	+	+	+	+

| TestTarget1 | 10.176.137.38 | TestParamsV2c | TestTag | 162	| 3	| 1500	| 32	|

+	+	+	+	+	+	+	+	+

+	SNMP VACM VIEW	+

| Viewtree Name | Subtree | Type	|

+	+	+	+

| cienaAll	| internet | include |

+	+	+	+

+------------ SNMP VACM GROUP MEMBER	+

| Group Name | Security Model | Security Name	|

+	+	+	+

| cienaGroup | v2c	| cienaSecurityV2c |

+	+	+

+

+

SNMP VACM GROUP ACCESS	+

| Group Name | Context | Context Match | Security Model | Security Level | Read View | Notify View |

+	+	+	+	+	+	+	+

| cienaGroup |	| exact	| v2c	| no-auth-no-priv |	| cienaAll	|

+	+	+	+	+	+	+	+

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

###### SNMP v2c Inform Configuration

SNMP v2c Inform:

      - Informs are same as Traps but the main difference is that the Informs are acknowledged back by the SNMP Manager.

      - SNMP Inform are enabled by specifying the type as ‘inform’ whereas type ‘trap’ will be used

for SNMP traps.

Procedure:

snmp community t1 security-name cienaSecurityV2c text-name cienaV2cCommunity snmp vacm view cienaAll include internet

snmp vacm group cienaGroup access "" v2c no-auth-no-priv context-match exact notify-view cienaAll

snmp vacm group cienaGroup member cienaSecurityV2c security-model v2c snmp target TestTarget1 target-params TestParamsV2c udp ip 10.176.137.38 snmp target-params TestParamsV2c v2c security-name cienaSecurityV2c snmp target TestTarget1 tag TestTag

snmp notify Test-NOTIFY1 type inform tag TestTag

Output:

- show snmp

+

SNMP NOTIFY	+

| Notification Name | Notification Tag | Notification Type |

+	+	+	+

| Test-NOTIFY1	| TestTag	| inform	|

+	+	+	+

+	SNMP COMMUNITY	+

| Community Index | Community Name	| Security Name	| Transport Tag |

+	+	+	+	+

| t1	| cienaV2cCommunity | cienaSecurityV2c |	|

+	+	+	+	+

+	SNMP TARGET-PARAMS	+

| Target Param Name | Security Name	| User Name | Security Model | Security Level |

+	+	+	+	+	+

| TestParamsV2c	| cienaSecurityV2c |	| v2c	| no-auth-no-priv |

+	+	+

+

+	+	+

SNMP TARGET	+

| Target Name | IP Address	| Param Name	| Tags	| UDP Port | Retry Count | Timeout | Prefix Length |

+	+	+	+	+	+	+	+	+

| TestTarget1 | 10.176.137.38 | TestParamsV2c | TestTag | 162	| 3	| 1500	| 32	|

+	+	+	+	+	+	+	+	+

+	SNMP VACM VIEW	+

| Viewtree Name | Subtree | Type	|

+	+	+	+

| cienaAll	| internet | include |

+	+	+	+

+------------ SNMP VACM GROUP MEMBER	+

| Group Name | Security Model | Security Name	|

+	+	+	+

| cienaGroup | v2c	| cienaSecurityV2c |

+	+	+

+

+

SNMP VACM GROUP ACCESS	+

| Group Name | Context | Context Match | Security Model | Security Level | Read View | Notify View |

+	+	+	+	+	+	+	+

| cienaGroup |	| exact	| v2c	| no-auth-no-priv |	| cienaAll	|

+	+	+	+	+	+	+	+

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

###### SNMP v2c Inform Configuration

SNMP v2c Inform:

      - Informs are same as Traps but the main difference is that the Informs are acknowledged back by the SNMP Manager.

      - SNMP Inform are enabled by specifying the type as ‘inform’ whereas type ‘trap’ will be used

for SNMP traps.

Procedure:

snmp community t1 security-name cienaSecurityV2c text-name cienaV2cCommunity snmp vacm view cienaAll include internet

snmp vacm group cienaGroup access "" v2c no-auth-no-priv context-match exact notify-view cienaAll

snmp vacm group cienaGroup member cienaSecurityV2c security-model v2c snmp target TestTarget1 target-params TestParamsV2c udp ip 10.176.137.38 snmp target-params TestParamsV2c v2c security-name cienaSecurityV2c snmp target TestTarget1 tag TestTag

snmp notify Test-NOTIFY1 type inform tag TestTag

Output:

- show snmp

+

SNMP NOTIFY	+

| Notification Name | Notification Tag | Notification Type |

+	+	+	+

| Test-NOTIFY1	| TestTag	| inform	|

+	+	+	+

+	SNMP COMMUNITY	+

| Community Index | Community Name	| Security Name	| Transport Tag |

+	+	+	+	+

| t1	| cienaV2cCommunity | cienaSecurityV2c |	|

+	+	+	+	+

+	SNMP TARGET-PARAMS	+

| Target Param Name | Security Name	| User Name | Security Model | Security Level |

+	+	+	+	+	+

| TestParamsV2c	| cienaSecurityV2c |	| v2c	| no-auth-no-priv |

+	+	+

+

+	+	+

SNMP TARGET	+

| Target Name | IP Address	| Param Name	| Tags	| UDP Port | Retry Count | Timeout | Prefix Length |

+	+	+	+	+	+	+	+	+

| TestTarget1 | 10.176.137.38 | TestParamsV2c | TestTag | 162	| 3	| 1500	| 32	|

+	+	+	+	+	+	+	+	+

+	SNMP VACM VIEW	+

| Viewtree Name | Subtree | Type	|

+	+	+	+

| cienaAll	| internet | include |

+	+	+	+

+------------ SNMP VACM GROUP MEMBER	+

| Group Name | Security Model | Security Name	|

+	+	+	+

| cienaGroup | v2c	| cienaSecurityV2c |

+	+	+

+

+

SNMP VACM GROUP ACCESS	+

| Group Name | Context | Context Match | Security Model | Security Level | Read View | Notify View |

+	+	+	+	+	+	+	+

| cienaGroup |	| exact	| v2c	| no-auth-no-priv |	| cienaAll	|

+	+	+	+	+	+	+	+

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

###### Create New User Account

Objective:

Setup a new user account to access the system.

Procedure:

- SAOS 10.1 supports two local user groups:

  - SYSTEM_ROLE_DIAG : The user has access to CLI, NETCONF, and also to Linux shell.

  - SYSTEM_ROLE_USER : The user has access to CLI and NETCONF.

- Enter the following command on CLI to create a new local user User1:

config

system aaa authentication users user User1 config username User1 role SYSTEM_ROLE_USER password Password1

- Verify that the user has been created:

5162_001> show aaa users

+	USER ACCOUNT TABLE	+

| Username | Role	| Sessions | Lockout |

+	+	+	+	+

| diag	| SYSTEM_ROLE_DIAG |	|	|

| user	| SYSTEM_ROLE_USER |	|	|

| User1	| SYSTEM_ROLE_USER |	|	|

+	+	+	+	+

- Ensure that the user has been added to NACM group -otherwise provisioning by local user will not be allowed.

  - config

  - nacm groups group super user-name User1

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

###### DNS Client Setup

Objective:

Objective is to verify setting up DNS client

Procedure:

- Enter the following command on CLI with an example DNS server 192.168.10.100:

config

oc-sys:system dns dns-client admin-status enabled

oc-sys:system dns dns-client domain-name ott.ciena.com oc-sys:system dns dns-client server 192.168.10.100

- You should see something like the following:

5170-010> show dns

+	DNS-CLIENT	+

| Name	| Value	|

+	+	+

| Admin-status	| enabled	|

| Domain-name	| ott.ciena.com |

| Domain-name-scope | user	|

| Server-scope	| user	|

+	+	+

+	USER-SERVERS	+

| IP Address	| Oper-status |

+	+	+

| 192.168.10.100 | enabled	|

+	+	+

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

###### ZTP

Objective:

Objective is to install SAOS 10.x with ZTP process. Network based ZTP process is useful when we want to ensure the same version of software is automatically installed on multiple nodes in the network. ZTP only allowed over VLAN 127, and untagged is not supported.

Procedure:

- Ensure you have DHCP environment setup is been done with either of

  - Option 66/67 to provide tftp server and filename of Command File

  - Option 125 to provide URL of Command Fil

- Ensure that the image you want to download is located on an HTTP/TFTP server

- Verify that the command file is located at the TFTP server and the config file is supplied

- Start the node if it is the first time it is being powered up

- Once the device boots up, Login to UI container using the management IP you checked in the previous step (default login/passwd displayed below):

ssh diag@x.x.x.x

username: diag password: ciena123

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

    - Verify Re-Install or Upgrade Software

Objective:

Objective is to verify downloading, installation, and activation of software.

Procedure:

- Load installation is through ONIE – Open Network Install Environment

- After initial installation of a SAOS load on a box, you should be able to upgrade to another load without need to go back to ONIE.

- *Following the example below and enter the following command to download the build

from user’s own URL – please note the URL below is for reference only:

software download url http://x.x.x.x/valimar-snapshot/manifest/saos-10-05-01- 0096.yml

- After the a few minutes, the build should have downloaded successfully. Execute a “show software” to check the download status.

- Once the build has downloaded successfully, execute the command install to install the software.

software install package saos-10-05-01-0096 defer-activation

- *Once installation is complete, activate the load to complete the upgrade.

software activate package saos-10-05-01-0096

- After the a few minutes, the build should have been activated successfully. Execute a

“show software” to check the status. You should see something like this:

5144-009> show software

| + ¦ | Name | ¦ | SOFTWARE STATE Value | + ¦ |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| + ¦ | Current operation | + ¦ | idle | + ¦ |  |  |
| ¦ ¦ ¦ ¦ + ¦ ¦ ¦ ¦ ¦ | RPC Status Running package version Package build info Active bootchain Components: BIOS image BOOT FPGA image ONIE image cn_alarm_1 | ¦ ¦ ¦ ¦ + ¦ ¦ ¦ ¦ ¦ | idle saos-10-05-01-0096 Wed Dec 02 03:16:15 2020 01-05-01-0096 0017 00.00.2a Ciena_2019.02.01-0040 01-05-01-0096 | autouser | onxvpnjk23 | ¦ ¦ ¦ ¦ + ¦ ¦ ¦ ¦ ¦ |
| ¦ | cn_central-logger_1 | ¦ | 01-05-01-0096 |  |  | ¦ |
| ¦ | cn_cfm_1 | ¦ | 01-05-01-0096 |  |  | ¦ |
| ¦ | cn_cnfp_1 | ¦ | 01-05-01-0096 |  |  | ¦ |
| ¦ | cn_collectd_1 | ¦ | 01-05-01-0096 |  |  | ¦ |
| ¦ | cn_dataplane_1 | ¦ | 01-05-01-0096 |  |  | ¦ |
| ¦ | cn_dhcp-ctrl_1 | ¦ | 01-05-01-0096 |  |  | ¦ |
| ¦ | cn_dhcpl3relay_1 | ¦ | 01-05-01-0096 |  |  | ¦ |
| ¦ | cn_dns_1 | ¦ | 01-05-01-0096 |  |  | ¦ |
| ¦ | cn_dot1x_1 | ¦ | 01-05-01-0096 |  |  | ¦ |
| ¦ | cn_eoam_1 | ¦ | 01-05-01-0096 |  |  | ¦ |
| ¦ | cn_erps_1 | ¦ | 01-05-01-0096 |  |  | ¦ |
| ¦ | cn_feds_1 | ¦ | 01-05-01-0096 |  |  | ¦ |
| ¦ | cn_hal_1 | ¦ | 01-05-01-0096 |  |  | ¦ |
| ¦ | cn_ipservices_1 | ¦ | 01-05-01-0096 |  |  | ¦ |
| ¦ | cn_lacp_1 | ¦ | 01-05-01-0096 |  |  | ¦ |
| ¦ | cn_lldp_1 | ¦ | 01-05-01-0096 |  |  | ¦ |
| ¦ | cn_ntp_1 | ¦ | 01-05-01-0096 |  |  | ¦ |
| ¦ | cn_pkix_1 | ¦ | 01-05-01-0096 |  |  | ¦ |
| ¦ | cn_pon-ctrl_1 | ¦ | 01-05-01-0096 |  |  | ¦ |
| ¦ | cn_ppm_1 | ¦ | 01-05-01-0096 |  |  | ¦ |

|	cn_rstp_1	| 01-05-01-0096	|

|	cn_sat_1	| 01-05-01-0096	|

|	cn_snmp-agent_1	| 01-05-01-0096	|

|	cn_storage_1	| 01-05-01-0096	|

|	cn_svcdirectory_1	| 01-05-01-0096	|

|	cn_sync-hal_1	| 01-05-01-0096	|

|	cn_sync_1	| 01-05-01-0096	|

|	cn_system_1	| 01-05-01-0096	|

|	cn_tdm_1	| 01-05-01-0096	|

|	cn_telemetry_1	| 01-05-01-0096	|

|	cn_twamp_1	| 01-05-01-0096	|

|	cn_ui_1	| 01-05-01-0096	|

|	cn_val-gui_1	| 01-05-01-0096	|

|	cn_ztp_1	| 01-05-01-0096	|

|	xg_xgrade-agent_1	| 01-05-01-0096	|

+	+	+

| Available packages:	|	|

|	saos-10-05-01-0096	| activated	|

+	+	+

*can combine the steps and use “software install” without the defer-activation parameter. “software install” will download, install and activate the load on the system in one step.

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

###### Verify Manual network-based installation

Objective:

Objective is to install SAOS 10.x on a system, using the manual network-based installation.

Procedure:

- Login to system console port.

- When ONIE comes up it will try to find the resource for ZTP. If you want to specify the location of the installer file manually, which is different from what it might fetch from network installer, you should stop it:

ONIE:/ # onie-discovery-stop

- Then proceed with the manual installation from the installer file from http, tftp or ftp server.

ONIE:/ # onie-nos-install http://x.x.x.x/installer.bin ONIE:/ # onie-nos-install tftp://x.x.x.x/installer.bin ONIE:/ # onie-nos-install ftp://x.x.x.x/installer.bin

Replace x.x.x.x with IP address or the URL of where the image is residing. Here is an example:

ONIE:/ # onie-nos-install http://x.x.x.x/valimar- snapshot/17-12-00-0141/meta-onie-installer-dnx/meta_01-00- 00-0181-core.bin

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

###### Verify USB-based installation of SAOS 10.x

Objective:

Objective is to install SAOS 10.x on a system using USB-based method.

Procedure:

- Ensure that the format of your USB stick is FAT32 formatted.

- Ensure that the image you want is on the USB stick

- Ensure that the image is named “onie-installer”. This is important. Currently, the load name cannot be named anything else.

- Start the node if it is the first time it is being powered up, or follow the following steps otherwise

- Login to UI container using the management IP you checked in the previous step:

ssh diag@x.x.x.x

username: diag password: ciena123

When ONIE comes up it will try to find the resource for ZTP. Wait for the installation to complete from here.

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

###### Verify NTP or verify the date and time

Objective:

Setting date and time are required before application of licenses. Set NTP or date for Ciena SAOS 10.x using Linux commands. Proper data and time setup are required for license application.

CLI NTP Procedure:

- Establish a ssh connection to ValCLI

ssh diag@X.X.X.X

password: ciena123

- Send the following config command

  - config

system ntp associations remote-ntp-server server-entry 10.33.80.21 admin-state enabled

- Check the NTP State with the following command

  - show ntp client

- The output would be something like:

+---------- NTP CLIENT STATE	+

| Name	|	Value |

+	+	+

| Admin State	| enabled	|

| Mode	| polling	|

| Polling Min Interval |	16	|

| Polling Max Interval |	16	|

| Auth Admin State	| disabled	|

| Synchronized	|	True	|

| Delay	|	62.161 |

| Offset	|	0.006 |

| Jitter	|	0.017 |

| Drift (PPM)	|	-4.485 |

+	+	+

+--------- NTP CONFIGURED SERVERS	+

| Address	| Auth Key ID | Admin State |

+	+	+	+

| 10.33.80.21 |	| enabled	|

+	+	+	+

+	NTP OPER SERVERS

+

| Address		| Auth Key ID	| Server State	| Server Condition | Auth State	| Offset |

+	+	+	+	+

+	+

| 10.33.80.21	|	| reach	| syspeer	| none

| 0.006 |

+	+	+	+	+

+	+

Date & Time Set Procedure:

- Login to UI using the management IP you checked in the previous step:

ssh diag@x.x.x.x

username: diag password: ciena123

- Enter the following command to change the date and time on the system. This will only work if NTP is disabled:

config

system set clock 2018-11-03T18:39:00Z

- Show the system time:

5162-002> show clock

+	System Clock	+

| Name	| Value	|

+	+	+

| Current Time | 2018-11-03 18:39:03 UTC |

- To enable NTP, the manually set clock will be overwritten.

config

system ntp admin-state enabled

5162-002> show clock

+	System Clock	+

| Name	| Value	|

+	+	+

| Current Time | 2018-11-08 13:22:28 UTC |

+	+	+

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

###### Layer 2 Control Protocol

Overview:

L2 control protocol (L2CP) allows the network operator to change the handling of L2 control frames such that they are discarded or forwarded as if they were data frames instead of being processed locally.

When an untagged L2CP frame is received on an ETTP, it is either trapped and sent to an L2 protocol for further processing, that is, peered, or it is treated as L2 data traffic, that is, classified as untagged data traffic. L2CP exposes a mechanism for modifying this default behavior, so that the L2CPs for certain protocols can be discarded or forwarded as L2 data traffic.

L2CP is supported on XGS platforms in SAOS 10.6

Scenario 1: L2CP Handling

Objective

The objective of this test is to create an L2CP profile and apply discard/forward dispositions to L2 protocols.

The default behavior for most Layer 2 control protocols in 10.X is to be forwarded as L2 data. In this scenario, we will apply a discard disposition to 802.1x, link-oam and Cisco-CDP protocols as an example on a 392X device. The control frames will be dropped at the UNI-port.

Procedure

- Create L2CP profile

  - l2cp-profiles l2cp-profile l2cpf1 protocol-disposition port-auth

untagged-disposition discard/forward protocol-disposition cisco-cdp untagged-disposition discard/forward protocol-disposition link-oam untagged-disposition discard/forward

- Attach an L2 control plane profile to a forwarding domain

  - fds fd FD1

  - mode vpls vlan-id 20(The FD is created without vlan-id on DNX)

  - l2cp-profile l2cpf1

- Create Untagged and L2CP classifier

  - classifiers classifier untagged filter-entry vtag-stack untagged-exclude-priority-tagged true

  - classifiers classifier class_l2cp filter-entry vtag-stack l2cp-exclude-priority true

- Create flow point and add the untagged and l2cp classifier to the flow-point

  - fps fp l2cp4 fd-name FD1 logical-port 4

classifier-list untagged classifier-list class_l2cp

ingress-l2-transform push-vid-400 vlan-stack 1 push-tpid tpid-8100 push-vid 20

exit exit

egress-l2-transform pop-vid-20vlan-stack 1 pop-type

Before applying the l2cp profile, L2CP frames sent from one test port are forwarded and received on the other port.

After applying the “discard disposition” under L2CP profile, we can see that L2 control frames are no longer received on the other side of the test set as they get dropped at the UNI-port of device.

3926-150> show ettps ettp 4 statistics

+	ETTP STATISTICS	+

| KEY	| VALUE |

+	+	+

| Name	| 4	|

|	|	|

|	In Bytes	| 2100 |

|	In Packets	| 30	|

|	In Broadcast Packets	| 0	|

|	Out Bytes	| 503	|

|	In Multicast Packets	| 30	|

| ¦ | Out Packets | ¦ | 3 | ¦ |
| --- | --- | --- | --- | --- |
| ¦ | Out Unicast Packets | ¦ | 0 | ¦ |
| ¦ | In Errors | ¦ | 0 | ¦ |
| ¦ | Out Errors | ¦ | 0 | ¦ |
| ¦ | Out Broadcast Packets | ¦ | 0 | ¦ |
| ¦ | In Undersize Packets | ¦ | 0 | ¦ |
| ¦ | Out Multicast Packets | ¦ | 3 | ¦ |
| ¦ | In 64 Octet Packets | ¦ | 0 | ¦ |
| ¦ | In 65 to 127 Octet Packets | ¦ | 30 | ¦ |
| ¦ | In 128 to 255 Octet Packets | ¦ | 0 | ¦ |
| ¦ | In 256 to 511 Octet Packets | ¦ | 0 | ¦ |
| ¦ | In 512 to 1023 Octet Packets | ¦ | 0 | ¦ |
| ¦ | In 1024 to 1518 Octet Packets | ¦ | 0 | ¦ |
| ¦ | In 1519 to 2047 Octet Packets | ¦ | 0 | ¦ |
| ¦ | In 2048 to 4095 Octet Packets | ¦ | 0 | ¦ |
| ¦ | In 4096 to 9216 Octet Packets | ¦ | 0 | ¦ |
| ¦ | Out 1519 to 2047 Octet Packets | ¦ | 0 | ¦ |

| ¦ | Out 2048 to 4095 Octet | Packets | ¦ | 0 | ¦ |
| --- | --- | --- | --- | --- | --- |
| ¦ | Out 4096 to 9216 Octet | Packets | ¦ | 0 | ¦ |
| ¦ | In Oversize Packets |  | ¦ | 0 | ¦ |
| ¦ | In Jabber Packets |  | ¦ | 0 | ¦ |
| ¦ | In Crc Error Packets |  | ¦ | 0 | ¦ |
| ¦ | Link Flap Events |  | ¦ | 0 | ¦ |
| ¦ | Out Discards |  | ¦ | 0 | ¦ |
| ¦ | In Unicast Packets |  | ¦ | 0 | ¦ |
| ¦ | In Discards Packets |  | ¦ | 30 | ¦ |
| + |  |  | + |  | + |

L2CP configuration can be displayed by executing the commands below.

3926-150> show flow-points flow-point l2cp4

+	FLOW POINT	+

| KEY	| VALUE	|

+	+	+

| Name	| l2cp4	|

| Forwarding Domain Name | FD1	|

| Logical Port	| 4	|

| MTU Size	| 2000	|

| Ingress L2 Transform	|	|

|	Ingress Name	| push-vid-20 |

|	Ingress VLAN Stack	|	|

|	Tag	| 1	|

|	Push TPID	| tpid-8100	|

|	Push VID	| 20	|

| Egress L2 Transform	|	|

|	Egress Name	| pop-vid-20 |

|	Egress VLAN Stack	|	|

|	Tag	| 1	|

|	Pop Type	| -	|

| Classifier List	|	|

|	| untagged	|

|	| class_l2cp |

+	+	+

+ FLOW POINT STATISTICS +

| KEY	| VALUE	|

+	+	+

| Name	| l2cp4	|

+	+	+

3926-150> show classifiers

+	CLASSIFIER	+

| Name	| Filter Parameter	|

+	+	+

| class_l2cp	| Classifier:untagged	|

| default-vid-127 | Classifier:tagged-all |

| untagged	| Classifier:untagged	|

+	+	+

3926-150> show l2cp-profiles l2cp-profile l2cpf1

+	L2CP PROFILE	+

| KEY	| VALUE	|

+	+	+

| Name	| l2cpf1	|

| Protocol Disposition	|	|

|	Protocol	| port-auth |

|	Untagged Disposition | discard	|

|	|	|

|	Protocol	| cisco-cdp |

|	Untagged Disposition | discard	|

|	|	|

|	Protocol	| link-oam |

|	Untagged Disposition | discard	|

+	+	+

Scenario 2: L2PT (Layer 2 Protocol Tunneling)

Layer 2 protocol tunneling is supported in 10.6 on DNX and XGS platforms. L2PT form requires that the frame’s original MAC DA get replaced with a configurable L2PT MAC address. In the reverse direction L2PT MAC address is replaced with proper IEEE or Cisco MAC DA.

Objective:

The objective of this test is to configure L2PT disposition for LACP, LACP Marker and LLDP as these protocols have a default “Peering” disposition. After provisioning the L2PT form, L2 protocol frames will be forwarded to the other test set instead of being processed on 392X device.

Procedure

- Create the L2PT MAC list with l2pt destination MAC

  - l2cp-profiles l2pt-macs 01:00:0C:CD:CD:D0

- Create the L2 control protocol profile

  - l2cp-profiles l2cp-profile l2cpf1

- Provision the forwarding-domain, classifiers and flow-point

  - fds fd FD1 mode vpls vlan-id 20 l2cp-profile l2cpf1

  - classifiers classifier untagged filter-entry vtag-stack untagged-exclude-priority-tagged true

  - classifiers classifier class_l2cp filter-entry vtag-stack l2cp-exclude-priority true

  - fps fp l2cp4fd-name FD1 logical-port 4 classifier-list untagged

classifier-list class_l2cp

ingress-l2-transform push-vid-20 vlan-stack 1 push-tpid tpid-8100 push-vid 20

exit exit

egress-l2-transform pop-vid-20 vlan-stack 1 pop-type

The steps below should be provisioned under the l2cp profile:

- Set the protocol in the protocol-disposition

  - protocol-disposition lldp

  - protocol-disposition lacp

  - protocol-disposition lamp

- Set the L2PT MAC address for the protocol for which L2PT forwarding needs to be enabled. This should be same MAC that is added in l2pt-macs list:

  - l2pt-mac 01:00:0C:CD:CD:D0

- Set the untagged disposition as “l2pt-forward”:

  - untagged-disposition l2pt-forward

After applying the “l2pt-forward disposition” under L2CP profile, we can see that L2 control frames (LACP/LLDP/LACP Marker) are now l received on the other side of the test without getting processed on the device.

L2CP configuration can be displayed by executing the commands below.

3926-150> show l2cp-profiles l2cp-profile l2cpf1

+	L2CP PROFILE	+

| KEY	| VALUE	|

+	+	+

| Name	| l2cpf1	|

| Protocol Disposition	|	|

|	Protocol	| lldp	|

|	Untagged Disposition | l2pt-forward	|

|	L2PT MAC Address	| 01:00:0c:cd:cd:d0 |

|	|	|

|	Protocol	| lacp	|

|	Untagged Disposition | l2pt-forward	|

|	L2PT MAC Address	| 01:00:0c:cd:cd:d0 |

|	|	|

|	Protocol	| lamp	|

|	Untagged Disposition | l2pt-forward	|

|	L2PT MAC Address	| 01:00:0c:cd:cd:d0 |

+	+	+

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

###### Configure G.8275.2 T-GM

Objective:

The objective of this test is to configure an G.8275.2 T-GM. A G.8275.2 T-GM configuration is similar to a 8275.1 T-GM, but It requires an IP interface to be configured.

Topology:

Antenna connected to the GNSS input (SMB antenna input).

Procedure:

The prerequisite for this test is to enable the GNSS.

- Set the PTP global parameters to a G.8275.2 profile and a GM clock type.

sync global network-option option-1 sync global reversion-mode revertive sync global wait-to-restore 0

sync ptp-global profile g.8275.2

sync ptp-global clock-type gm sync ptp-global protocol-version 2

sync ptp-global timestamp-mode one-step sync ptp-global max-global-session 44 sync ptp-global dscp 41

sync ptp-global ttl 255

- Configure an IP interface. Note: The interface used below in on logical port 2.

classifiers classifier vlan501 filter-entry vtag-stack vtags 1 vlan-id 501 fds fd VLAN501 mode vpls

fps

fp VLAN501

stats-collection on fd-name VLAN501 logical-port 2

classifier-list-precedence 7 classifier-list vlan501

mtu-size 9216

egress-l2-transform push-vid-501 vlan-stack 1

push-tpid tpid-8100 push-vid 501

exit exit exit Exit

oc-if:interfaces interface if2 config name if2 vrfName default cn-if:type ip mtu 1500 admin-status true underlay-binding config fd VLAN501

oc-if:interfaces interface if2 ipv4 addresses address 20.20.20.1 config ip

20.20.20.1 prefix-length 24

- Configure PTP outputs.

sync output-references ptp-output-reference PTP-Out interface if2 sync output-references ptp-output-reference PTP-Out max-grant-rate 64

sync output-references ptp-output-reference PTP-Out signalling on

Expected Results:

PTP outputs should be active.

show sync ptp output-references

+------------ SYNC PTP OUTPUT REFERENCES	+

| Name	| Interface | Oper State | PTP Port State |

+	+	+	+	+

| PTP-Out |	if2 |	Up	|	Master	|

+	+	+	+	+

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

    - MEP Support on QinQ

CFM/Y.1731 PDU’s with l2-transform on MEP and flow-point

Outgoing CFM or y.1731 pdu’s can be untagged, single tagged or double tagged. Tag on packet is determined by l2-transform configured on the MEP and egress/ingress transform configured on the fp on which the MEP is created.

From 10.7.0 onwards, all types of ingress and egress transforms will be applicable to the UP

MEP. The packet format for all types of CFM PDU’s will be same as the data traffic.

Xform on MEP : Dual-Tagged

The input packet format with MEP transform applied from CFM application :

| DA - SA | TAG1 | TAG2 | CFM PDU |
| --- | --- | --- | --- |

Example configuration for l2-transfrom on the MEP(double-tagged).

Objective:

The objective of this test is to create UP MEP L2-transform with double tag.

Topology:

Procedure:

Please note the configuration below highlights the configuration on one of the nodes 517x-1 from the above-mentioned topology

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

###### Verification

Test Case Results:

Passed: Yes 	Verified by 	Date/Time		 Comments

###### MPLS-TP Static Tunnel – LSP BFD – Hardware acceleration

Objective:

Validate that LSP BFD session can be enabled on MPLS-TP tunnel. Also test hardware acceleration on the LSP BFD session. Only supported on 51xx devices.

Procedure:

- Create static MPLS-TP tunnel with LSP BFD session – hardware acceleration disabled.

  - config

## Ingress Node

tp-corouted-tunnels tunnel to5144-20-b 192.168.44.236 192.168.44.237

tunnel-type static-ingress-corouted forward-out-segment forward-out-label 30

forward-out-segment next-hop-ip 172.16.4.2

reverse-in-segment reverse-in-label 31

source-tunnel-id 30

destination-tunnel-id 31

bfd-cfg-parms admin-down false bfd-cfg-parms accelerate false exit

exit

##Egress Node

tp-corouted-tunnels tunnel to5144-20-b 192.168.44.236 192.168.44.237

tunnel-type static-egress-corouted reverse-out-segment reverse-out-label 31

reverse-out-segment prev-hop-ip 172.16.4.1

forward-in-segment forward-in-label 30

source-tunnel-id 30

destination-tunnel-id 31 bfd-cfg-parms admin-down false bfd-cfg-parms accelerate false exit

exit

- Validate that the BFD session is not hardware accelerated, and the MPLS-TP tunnel is operational

Local Node

- show tp-tunnel-corouted

Remote Node

  - show bfd sessions Local Node

Remote Node

- Set the hardware acceleration to enabled – must admin-disable the BFD session first.

##Ingress Node

tp-corouted-tunnels tunnel to5144-20-b 192.168.44.236 192.168.44.237

bfd-cfg-parms statistics-collection on

bfd-cfg-parms admin-down true bfd-cfg-parms accelerate true

bfd-cfg-parms required-min-rx-interval 3300

bfd-cfg-parms desired-min-tx-interval 3300 apply

bfd-cfg-parms admin-down false

apply

exit exit

##Egress Node

tp-corouted-tunnels tunnel to5144-20-b 192.168.44.236 192.168.44.237

tunnel-type static-egress-corouted

bfd-cfg-parms statistics-collection on

bfd-cfg-parms admin-down true bfd-cfg-parms accelerate true

bfd-cfg-parms required-min-rx-interval 3300

bfd-cfg-parms desired-min-tx-interval 3300 apply

bfd-cfg-parms admin-down false

apply exit exit

- Validate that the MPLS-TP tunnel and the hardware accelerated bfd session are up.

  - show tp-tunnel-corouted

Local Node:

Remote Node:

>show bfd sessions

Local Node:

Remote Node:

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

###### Dynamic PWE – WTR

Objective:

Verify provisioning of WTR parameter.

Procedure:

- Go to the configuration terminal and provision the protected PWE.

  - config

pseudowires pseudowire PW1 mode spoke protect true role primary configured-pw peer-ip 192.168.28.24 pw-id 100

pseudowires pseudowire bkPW1 mode spoke protect true role backup primary-pw PW1 configured-pw peer-ip 192.168.28.78 pw-id 200

l2vpn-services l2vpn service1 signaling-type ldp pseudowire PW1

pseudowire bkPW1 forwarding-domain hvpls exit

exit

- Validate the PWE, note that PWE reversion is not set – ie: no reversion – even though the WTR is set to 30sec by default.

5144-0019-Node4> show pseudowires pseudowire PW1

+	PSEUDOWIRES STATE

+

| KEY	| VALUE	|

+	+

+

| PW Id	| 100	|

| Name	| PW1	|

| Peer IP	| 192.168.28.24	|

| Oper State	| DOWN	|

| In Label	| 52002	|

| Out Label	| 52000	|

| Mode	| Spoke	|

| PW Role	| Primary	|

| PW Type	| Dynamic	|

| Service Type	| ethernet	|

| Protection State	| passive	|

| Remote PW Status	| pseudowire-forwarding	|

| Local PW Status	| pseudowire-not-forwarding |

| Statistics Collection	| Off	|

| PW Load Balance	| disabled	|

| Operational Fat	| None	|

| Configured CW	| non-preferred	|

| Operational CW	| OFF	|

| Operational CC Type	| cctype-4	|

| MTU	| 1500	|

| Tunnel Type	| OSPF-SR	|

| Reversion Hold Time	| 30 sec	|

| Configured PW-Reversion	| none	|

| Manual Switchover	| False	|

| Ignore MTU Mismatch	| False	|

| MS-PW ID	| NONE	|

| MAC Withdraw	| True	|

| PW Fault Reason	| no-fault	|

| PW Status Local Refresh Timer | 600 sec	|

+	+

+

- Configure the primary PWE so that the PWE reversion is enabled, and the Reversion Hold Time (WTR) is set to 300sec.

pseudowires pseudowire PW1 pw-reversion enabled pseudowires pseudowire PW1 reversion-hold-time 300

- Validate pseudowire, now pseudowire will revert to primary PWE when the fault recovers and the WTR hold timer expires.

5144-0019-Node4> show pseudowires pseudowire PW1

+	PSEUDOWIRES STATE	+

| KEY	| VALUE	|

+	+	+

| PW Id	| 100	|

| Name	| PW1	|

| Peer IP	| 192.168.28.24	|

| Oper State	| UP	|

| In Label	| 52002	|

| Out Label	| 52000	|

| Mode	| Spoke	|

| PW Role	| Primary	|

| PW Type	| Dynamic	|

| Service Type	| ethernet	|

| Protection State	| active	|

| Remote PW Status	| pseudowire-forwarding |

| Local PW Status	| pseudowire-forwarding |

| Statistics Collection	| Off	|

| PW Load Balance	| disabled	|

| Operational Fat	| None	|

| Configured CW	| non-preferred	|

| Operational CW	| OFF	|

| ¦ | Operational CC | Type | ¦ |  | cctype-4 | ¦ |
| --- | --- | --- | --- | --- | --- | --- |
| ¦ ¦ ¦ | MTU Tunnel Type Reversion Hold | Time | ¦ ¦ ¦ |  | 1500 OSPF-SR 300 sec | ¦ ¦ ¦ |
| ¦ ¦ ¦ ¦ ¦ ¦ ¦ + | Configured PW-Reversion Manual Switchover Ignore MTU Mismatch MS-PW ID MAC Withdraw PW Fault Reason PW Status Local Refresh |  | Timer | ¦ ¦ ¦ ¦ ¦ ¦ ¦ + | enabled False False NONE True no-fault 600 sec | ¦ ¦ ¦ ¦ ¦ ¦ ¦ + |

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

###### FAT PW Configuration

Objective: Objective of this section is to provide a breakdown of step by step configuration on each node that needs to be performed to enable FAT PW feature.

- LAG configuration

- Forwarding domain, flow points configuration

- IP interfaces configuration

- IGP such as ISIS or OSPF configuration (we are using ISIS L1 for our test purpose)

- LDP & MPLS configuration

- Targeted LDP and pseudo wire configuration

- FAT capability enable within pseudo wire context

- L2VPN service creation, associating it to attachment circuit & pseudowire

- Attachment circuit creation to transmit CE traffic over L2VPN infrastructure

Procedure:

5170-0037 (PE1) Node:

config

# LAG Configuration

oc-if:interfaces interface PE1-P1-LAG config name PE1-P1-LAG cn- if:type lag

oc-if:interfaces interface PE1-P1-LAG config agg agg-admin-mode agg-mode-lacp

oc-if:interfaces interface PE1-P1-LAG config agg member-ports 17 oc-if:interfaces interface PE1-P1-LAG config agg member-ports 18 exit

exit exit exit exit

oc-if:interfaces interface PE1-P1-LAG config agg member-ports 18 exit

exit

exit exit exit

# NNI Port Classifier

classifiers classifier vlan101 filter-entry vtag-stack vtags 1 vlan-id 101

# FD

fds fd fat-pw-fd mode vpls

# FP

fps

fp fat-pw-fp

fd-name fat-pw-fd logical-port PE1-P1-LAG stats-collection on

classifier-list-precedence 7 classifier-list vlan101

mtu-size 9216

egress-l2-transform push-vid-101 vlan-stack 1

push-tpid tpid-8100 push-vid 101

exit exit exit exit

# IP Interfaces

oc-if:interfaces interface PE1-P1-LAG-intr config name PE1-P1-LAG- intr cn-if:type ip admin-status true mtu 1500 underlay-binding config fd fat-pw-fd

oc-if:interfaces interface PE1-P1-LAG-intr ipv4 addresses address

11.11.11.1 config ip 11.11.11.1 prefix-length 24

oc-if:interfaces interface lb1 config name lb1 cn-if:type loopback

oc-if:interfaces interface lb1 ipv4 addresses address 37.37.37.1 config ip 37.37.37.1 prefix-length 32

# ISIS Config

isis instance 100 net 49.BBBB.0370.3703.7001.00 isis instance 100 level-type level-1

isis instance 100 interfaces interface lb1 exit

exit

interfaces interface PE1-P1-LAG-intr

exit exit exit exit

# ISIS Interfaces

isis instance 100 interfaces interface PE1-P1-LAG-intr level-type level-1

isis instance 100 interfaces interface lb1 level-type level-1

# LDP

ldp instance default lsr-id 37.37.37.1

ldp instance default interfaces interface lb1 enable-ipv4 true

ldp instance default interfaces interface PE1-P1-LAG-intr enable- ipv4 true

# MPLS Label Switching

mpls interfaces interface lb1 label-switching true

mpls interfaces interface PE1-P1-LAG-intr label-switching true

# Targeted LDP for PW

ldp instance default target-ldp peers 39.39.39.1 exit

exit exit exit

# VPWS Dynamic PW

pseudowires

pseudowire pe1_pe2_spoke_pw1 mode spoke

pw-loadbalance fat-pw fat-capability tx-rx configured-pw

pw-id 1

peer-ip 39.39.39.1 exit

exit exit

# UNI Port

# Classifier

classifiers classifier vlan102 filter-entry vtag-stack vtags 1 vlan-id 102

# FD

fds fd AC_UNI_FD1 mode vpws

# FP

fps

fp AC_UNI_FP1 stats-collection on fd-name AC_UNI_FD1 logical-port 10

mtu-size 2000

cos-to-frame-map default-c2f frame-to-cos-map default-f2c classifier-list-precedence 102 classifier-list vlan102

exit exit

# L2VPN

l2vpn-services l2vpn L1

forwarding-domain AC_UNI_FD1 pseudowire pe1_pe2_spoke_pw1 service-type vlan

signaling-type ldp mtu 1500

exit exit

#REMOVE L2vpn services (this config is shared for reference only if end user wants to remove L2VPN)

config t

no l2vpn-services no pseudowires

no ldp instance default target-ldp peers 4.4.4.4

5170-0038 (P1) Node:

#P1:

# LAG Configuration

config

oc-if:interfaces interface PE1-P1-LAG config name PE1-P1-LAG cn-if:type lag

oc-if:interfaces interface PE1-P1-LAG config agg agg-admin-mode agg-mode- lacp

oc-if:interfaces interface PE1-P1-LAG config agg member-ports 17 oc-if:interfaces interface PE1-P1-LAG config agg member-ports 18 exit

exit exit exit

oc-if:interfaces interface PE1-P1-LAG config agg member-ports 18

exit exit exit exit exit

# NNI Port Classifier

classifiers classifier vlan101 filter-entry vtag-stack vtags 1 vlan-id 101

# FD

fds fd fat-pw-fd mode vpls

# FP

fps

fp fat-pw-fp

fd-name fat-pw-fd logical-port PE1-P1-LAG stats-collection on

classifier-list-precedence 7 classifier-list vlan101

mtu-size 9216

egress-l2-transform push-vid-101 vlan-stack 1

push-tpid tpid-8100 push-vid 101

exit exit exit exit

# IP Interfaces

oc-if:interfaces interface PE1-P1-LAG-intr config name PE1-P1-LAG-intr cn- if:type ip admin-status true mtu 1500 underlay-binding config fd fat-pw- fd

oc-if:interfaces	interface	PE1-P1-LAG-intr	ipv4	addresses	address

11.11.11.2 config ip 11.11.11.2 prefix-length 24

oc-if:interfaces interface lb1 config name lb1 cn-if:type loopback

oc-if:interfaces interface lb1 ipv4 addresses address 38.38.38.1 config ip 38.38.38.1 prefix-length 32

#P1 to PE2 config part # LAG Configuration config

oc-if:interfaces interface P1-PE2-LAG config name P1-PE2-LAG cn-if:type lag

oc-if:interfaces interface P1-PE2-LAG config agg agg-admin-mode agg-mode- lacp

oc-if:interfaces interface P1-PE2-LAG config agg member-ports 37 oc-if:interfaces interface P1-PE2-LAG config agg member-ports 38 exit

exit exit exit exit

# NNI Port Classifier

classifiers classifier vlan90 filter-entry vtag-stack vtags 1 vlan-id 90

# FD

fds fd fat-pw-fd-2 mode vpls

# FP

fps

fp fat-pw-fp-2

fd-name fat-pw-fd-2 logical-port P1-PE2-LAG stats-collection on

classifier-list-precedence 7 classifier-list vlan90

mtu-size 9216

egress-l2-transform push-vid-90 vlan-stack 1

push-tpid tpid-8100 push-vid 90

exit exit exit exit

# IP Interfaces

oc-if:interfaces interface P1-PE2-LAG-intr config name P1-PE2-LAG-intr cn- if:type ip admin-status true mtu 1500 underlay-binding config fd fat-pw- fd-2

oc-if:interfaces	interface	P1-PE2-LAG-intr	ipv4	addresses	address

10.10.11.1 config ip 10.10.11.1 prefix-length 24

# ISIS Config

isis instance 100 net 49.BBBB.0380.3803.8001.00 isis instance 100 level-type level-1

isis instance 100 interfaces interface lb1 exit

exit

interfaces interface PE1-P1-LAG-intr

exit exit exit

interfaces interface P1-PE2-LAG-intr exit

exit exit exit

# ISIS Interfaces

isis instance 100 interfaces interface PE1-P1-LAG-intr level-type level-1 isis instance 100 interfaces interface P1-PE2-LAG-intr level-type level-1 isis instance 100 interfaces interface lb1 level-type level-1

# LDP

ldp instance default lsr-id 38.38.38.1

ldp instance default interfaces interface lb1 enable-ipv4 true

ldp instance default interfaces interface PE1-P1-LAG-intr enable-ipv4 true ldp instance default interfaces interface P1-PE2-LAG-intr enable-ipv4 true

# MPLS Label Switching

mpls interfaces interface lb1 label-switching true

mpls interfaces interface PE1-P1-LAG-intr label-switching true mpls interfaces interface P1-PE2-LAG-intr label-switching true

5170-0039 (PE2) Node:

# LAG Configuration

config

oc-if:interfaces interface P1-PE2-LAG config name P1-PE2-LAG cn-if:type lag

oc-if:interfaces interface P1-PE2-LAG config agg agg-admin-mode agg-mode- lacp

oc-if:interfaces interface P1-PE2-LAG config agg member-ports 17 oc-if:interfaces interface P1-PE2-LAG config agg member-ports 18 exit

exit exit exit exit

# NNI Port Classifier

classifiers classifier vlan90 filter-entry vtag-stack vtags 1 vlan-id 90

# FD

fds fd fat-pw-fd-2 mode vpls

# FP

fps

fp fat-pw-fp-2

fd-name fat-pw-fd-2 logical-port P1-PE2-LAG stats-collection on

classifier-list-precedence 7 classifier-list vlan90

mtu-size 9216

egress-l2-transform push-vid-90 vlan-stack 1

push-tpid tpid-8100 push-vid 90

exit exit exit exit

# IP Interfaces

oc-if:interfaces interface P1-PE2-LAG-intr config name P1-PE2-LAG-intr cn- if:type ip admin-status true mtu 1500 underlay-binding config fd fat-pw- fd-2

oc-if:interfaces  interface  P1-PE2-LAG-intr  ipv4  addresses  address

10.10.11.2 config ip 10.10.11.2 prefix-length 24

oc-if:interfaces interface lb1 config name lb1 cn-if:type loopback

oc-if:interfaces interface lb1 ipv4 addresses address 39.39.39.1 config ip 39.39.39.1 prefix-length 32

# ISIS Config

isis instance 100 net 49.BBBB.0390.3903.9001.00 isis instance 100 level-type level-1

isis instance 100 interfaces interface lb1 exit

exit

interfaces interface P1-PE2-LAG-intr exit

exit exit exit

# ISIS Interfaces

isis instance 100 interfaces interface P1-PE2-LAG-intr level-type level-1 isis instance 100 interfaces interface lb1 level-type level-1

# LDP

ldp instance default lsr-id 39.39.39.1

ldp instance default interfaces interface lb1 enable-ipv4 true

ldp instance default interfaces interface P1-PE2-LAG-intr enable-ipv4 true

# MPLS Label Switching

mpls interfaces interface lb1 label-switching true

mpls interfaces interface P1-PE2-LAG-intr label-switching true

# Targeted LDP for PW

ldp instance default target-ldp peers 37.37.37.1

exit exit exit exit

# VPWS Dynamic PW

pseudowires

pseudowire pe1_pe2_spoke_pw1 mode spoke

pw-loadbalance fat-pw fat-capability tx-rx configured-pw

pw-id 1

peer-ip 37.37.37.1 exit

exit exit

# UNI Port

# Classifier

classifiers classifier vlan103 filter-entry vtag-stack vtags 1 vlan-id 103

# FD

fds fd AC_UNI_FD1 mode vpws

# FP

fps

fp AC_UNI_FP1 stats-collection on fd-name AC_UNI_FD1 logical-port 10

mtu-size 2000

cos-to-frame-map default-c2f frame-to-cos-map default-f2c classifier-list-precedence 103 classifier-list vlan103

exit exit

# L2VPN Service

l2vpn-services l2vpn L1

forwarding-domain AC_UNI_FD1 pseudowire pe1_pe2_spoke_pw1 service-type vlan

signaling-type ldp mtu 1500

exit exit

#REMOVE L2vpn services (this config is shared for reference only if end user wants to remove L2VPN)

config t

no l2vpn-services no pseudowires

no ldp instance default target-ldp peers 4.4.4.4

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

    - FAT PW Verification & Results

Objective:

Objective of this section is to provide a cli commands for verification of FAT PW status & results analysis. We will be using various alternate methods to verify, however at the moment there is no direct cli command available in 10.5.1 to verify FAT PW status. We will capture cli outputs from PE nodes PW.

Procedure:

On PE nodes

Verify PW status:

Ping response of PW

Verify FAT signalling enable on node:

- On PE1 node the output of ettps shows an even spread of traffic on ingress direction of LAG ports 20 & 21, for traffic originated by ixia connected for pt 1 5 and destined towards ixia connected port 1 at PE2.

This uniform distribution of traffic along with wireshark snapshot of packet capture proves that 10x node is hashing traffic on two links of LAG for a L2VPN VPLS.

PE 1 ETTP stats

PE 2 ETTP stats

- Ingress Packet Capture taken on port 20 of PE2 node which shows the of FAT PW label (label 360 can be seen generated)

- For details of hashing algorithm works please refer to FAT PW Signaling capability 10.5 TOI topic, however in our test setup we are using ixia to create two different source MACs, which separates different traffic flows, essentially resulting in FAT label generation and efficient hashing. Snapshot setting from Ixia can be seen below

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

###### TDM PWE – OC3/STM1

Objective:

- Validate OC3 TDM PWE for pluggable XCVR-GES101. The PWE is between 10.5.1 and 6.20.

Procedure:

- On the SAOS 6.20 node, 3926_0060, port 6 is connected to OC3 fiber loopback.

- On the SAOS 10.5.1 node, 5144-009, port 1 is connected to a TDM OC3 testset.

- TDM pluggable used in both 5144 and 3926 is XCVR-GES101.

- Find the mac address of the SAOS 6.20 port 6 by doing a port show. Keep this mac address handy.

CN3926_0060> port show port 6

+

PORT 1.6 INFO	+

| Field	| Admin	| Oper	|

+	+	+	+

| Type	| CSOP OC3/STM1	|

| Description	|	|

| Spanning Tree State	| Forwarding	|

| MAC Address	| 74:87:bb:6f:70:87	|

| Phy Loopback	| Off	|

| Facility Loopback	| Off	|

| Link Flap Detection	| Off	|

| Link Flap Count	| 5	|

| Link Flap Detect Time	| 10	|

| Link Flap Hold Time	| 300	|

| Hold-Off	| Disabled	|

| Hold-Off Time (ds)	| 10	|

| Flow Loop Detect State	| Off	|

| Flow Loop Detection	| Off	| Off	|

| Y-Protection state	| none	|

| L2CFT Status	| Disabled	| Disabled	|

| L2CFT Profile	|	|	|

| Link State	| Enabled	| Enabled	|

| VLLI State	|	| Enabled	|

| Rx Low Power Detection	|	|	|

| State Group Link State	|	|	|

| Degrade Detection Mode	| off	| off	|

| Degrade State	|	| None	|

| Mode	| sfp	| sfp	|

| Speed	| 155.52 Mbits/s	| 1 Gbps	|

| Duplex	| full	| full	|

| Flow Control	| off	| off	|

| Auto Negotiation	| Enabled	| Enabled	|

| Flow Control Advertised | off	|	|

| ¦ ¦ | Speed Advertised Duplex Advertised | ¦ ¦ | ¦ ¦ | ¦ ¦ |
| --- | --- | --- | --- | --- |
| ¦ | PVID | ¦ 1 | ¦ 1 | ¦ |
| ¦ | Untag Ingress Data Vid | ¦ 0 | ¦ 0 | ¦ |
| ¦ | Fixed Resolved CoS | ¦ 0 | ¦ 0 | ¦ |
| ¦ | Fixed Resolved Color | ¦ green | ¦ green | ¦ |
| ¦ | Ingress VLAN Filter | ¦ Enabled | ¦ Enabled | ¦ |
| ¦ | Ingress VS Filter | ¦ Off | ¦ Off | ¦ |
| ¦ | Egress VS Filter | ¦ Off | ¦ Off | ¦ |
| ¦ | Acceptable Frame Type | ¦ VLAN tagged only | ¦ VLAN tagged only | ¦ |
| ¦ | Egress Untag VLAN | ¦ 1 | ¦ 1 | ¦ |
| ¦ | Max Frame Size | ¦ 1526 | ¦ 1526 | ¦ |
| ¦ | Inter-Packet Gap Size | ¦ 12 | ¦ 12 | ¦ |
| ¦ ¦ ¦ ¦ | Egress Frame Cos Policy Untagged Data VS Untagged Ctrl VS VS L2 Transform Mode | ¦ ignore ¦ ¦ ¦ i-push,e-pop | ¦ ignore ¦ ¦ | ¦ ¦ ¦ ¦ |
| ¦ | Resolved CoS Policy | ¦ dot1d-tag1-cos | ¦ dot1d-tag1-cos | ¦ |
| ¦ | Service Port Type | ¦ Subscriber | ¦ Subscriber | ¦ |
| ¦ | Eth VC EtherType | ¦ 8100 | ¦ 8100 | ¦ |
| ¦ | Eth VC EtherType Policy | ¦ all | ¦ all | ¦ |
| ¦ | Mirror-port | ¦ Off | ¦ Off | ¦ |
| ¦ | Mirroring Encapsulation | ¦ none | ¦ | ¦ |
| ¦ | Mirror Encap VID | ¦ 1 | ¦ | ¦ |
| ¦ ¦ ¦ ¦ | Mirror Encap TPID Ingress-mirror Egress-mirror Ingress to Egress QMap | ¦ 8100 ¦ ¦ ¦ Default-RCOS | ¦ ¦ ¦ ¦ Default-RCOS | ¦ ¦ ¦ ¦ |
| ¦ ¦ | XCVR caps mismatch Ingress FCOS->RCOS Map | ¦ [NONE] ¦ DefaultFcosRcos | ¦ DefaultFcosRcos | ¦ ¦ |
| ¦ | Ingress RCOS Remark L2 | ¦ False | ¦ False | ¦ |
| ¦ | Egress RCOS->FCOS Map | ¦ DefaultRcosFcos | ¦ DefaultRcosFcos | ¦ |
| ¦ ¦ ¦ ¦ ¦ + ¦ ¦ | Benchmark State Ingress ACL Egress Capture Ingress Capture System-Rsvd-Port Status Aggregation Membership VLAN Membership | ¦ Disabled ¦ ¦ ¦ ¦ Disabled + ¦ ¦ 1,127 | ¦ Disabled ¦ ¦ ¦ + ¦ ¦ 1,127 | ¦ ¦ ¦ ¦ ¦ + ¦ ¦ |
| + |  | + | + | + |

- Disable all ethernet protocols on port 6 on 3926-0060.

lldp set port 6 mode disable rstp disable port 6

mstp disable port 6

- Provision the pluggable on the SAOS 10.5.1 device 5144-009.

config

tdm-global mode ansi ctpid 0x8100 tdm-global sfp-auto-upgrade true

no fps fp remote-fp2 no fps fp remote-fp1 no fps fp remote-fp26

xcvrs xcvr 1 properties mode OC3-STM1

tdm-ports tdm-port 1 clock-mode internal tdm-ports tdm-port 1 ettp-id 1

tdm-ports tdm-port 1 type smart-sfp

tdm-ports tdm-port 1 port-mode oc3-stm1 tdm-ports tdm-port 1 src-address port

- Find the SAOS 10x pluggable mac address by displaying the tdm port.	Keep this mac address handy.

5144-009> show tdm ports port 1

+----------------- TDM PORT 1 CONFIGURATION AND STATUS INFORMATION	+

| Parameter	| Value	|

+	+	+

| TDM-Port-Name	| 1	|

| Type	| Smart-Sfp	|

| Port-Mode	| Oc3-Stm1	|

| Parent-Id	| 1	|

| Ettp-Id	| 1	|

| Src-Address	| Port	|

| Clock-Mode	| Internal	|

| Loopback-Mode	| None	|

| Loopback-Type	| None	|

| Loopback-Channel	| None	|

| Prbs-Insert	| None	|

| Prbs-Insert-Channel	| None	|

| Prbs-Monitor	| None	|

| Prbs-Monitor-Channel	| None	|

| Section-Trace	| Not Set - Using Default Pattern |

| Signal-Fail	| None	|

| Signal-Degrade	| None	|

+	+	+

| Operational Information	|	|

+	+	+

| Oper-State	| Down	|

| Oper-Category	| Oc3	|

| Source-Mac	| 18:92:A4:96:E6:89	|

| Firmware-Oper-State	| Upgrade-Complete	|

| Packet-Processor-Oper-State	| Upgrade-Complete	|

| Active-Alarms	| Los Ais Lof	|

| Channels-Assigned	| None	|

| Code-Violations-line	| 0	|

| Errored-Seconds-Line	| 0	|

| Severely-Errored-Seconds-Line	| 0	|

| Unavailable-Seconds-Line	| 5767	|

| Ais-Seconds-Line	| 0	|

| Los-Seconds-Section	| 5741	|

| Code-Violations-Section	| 0	|

| ¦ | Errored-Seconds-Section | ¦ | 5759 | ¦ |
| --- | --- | --- | --- | --- |
| ¦ | Severely-Errored-Seconds-Section | ¦ | 5759 | ¦ |
| ¦ | Severely-Errored-Frame-Seconds-Section | ¦ | 5741 | ¦ |
| ¦ | Loopback-Code-Violations-Line | ¦ | 0 | ¦ |
| ¦ | Loopback-Errored-Seconds-Line | ¦ | 0 | ¦ |
| ¦ | Loopback-Severely-Errored-Seconds-Line | ¦ | 0 | ¦ |
| ¦ | Loopback-Unavailable-Seconds-Line | ¦ | 0 | ¦ |
| ¦ | Loopback-Ais-Seconds-Line | ¦ | 0 | ¦ |
| ¦ | Loopback-Los-Seconds-Section | ¦ | 0 | ¦ |
| ¦ | Loopback-Code-Violations-Section | ¦ | 0 | ¦ |
| ¦ | Loopback-Errored-Seconds-Section | ¦ | 0 | ¦ |
| ¦ | Loopback-Severely-Errored-Seconds-Section | ¦ | 0 | ¦ |
| ¦ | Loopback-Severely-Errored-Frame-Seconds-Section | ¦ | 0 | ¦ |
| + |  | + |  | + |

5144-009>

- Config the following on SAOS 6.20 OC3 TDM port 6.

vlan add vlan 2200 port 1.4  add NNI Port 1.4 into VLAN

virtual-circuit ethernet create vc vcEth2 vlan 2200

port tdm oc3-stm1 create oc3-stm1 oc3P6 port 6 mac port-mac 

Create tdm OC3 SFP port

port tdm enable port oc3P6

attachment-circuit satop create ac ac-oc3P6 port oc3P6 rtp off channel 0 channel-type ds1-e1  satop encapsulation supported, rtp should be set to off

virtual-circuit tdm-mef8 create vc vc-oc3P6 ac ac-oc3P6 peer- mac 18:92:A4:96:E6:89 in-ecn 2201 out-ecn 2200	 peer mac is the 10x TDM port mac

virtual-switch cross-connect create xc xc-10x-oc3test tdm-vc vc-oc3P6 eth-vc vcEth2

- Config the following on SAOS 10.5.1. TDM OC3 pluggable is inserted into port 1.

classifiers classifier v2200 filter-entry vtag-stack vtags 1 vlan-id 2200

fds fd v2200 mode tdm-vpws

fps fp fp26-v2200 logical-port 26 fd-name v2200 classifier-list v2200

fps fp fp1-v2200 logical-port 1 fd-name v2200 classifier-list v2200

# TDM pseudowires ECID must match with remote node, mac address is the SAOS 6x tdm01 mac displayed previously

tdm-pseudowires tdm-pseudowire tdm2200 emulation-type satop encapsulation mef8 in-ecid 2200 out-ecid 2201 tdm-vid 2200 peer-mac 74:87:bb:6f:70:87 parent-tdm-port 1 channel-type ds1- e1 rtp off channel 0

tdm-pseudowires tdm-pseudowire tdm2200 jitter-buffer 3000 tdm-pseudowires tdm-pseudowire tdm2200 admin-state enabled

- Verify OC3 traffic on the testset connected to the 10x port 2 and 6.20 TDM port 6 with fiber loopback.

- With TDM traffic, ensure that synchronization is provisioned and that both 10x and 6x devices are synced to a singled timing source.

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

###### Add a COS to frame map to a flow point

Objective:

Objective is to verify binding of a COS to frame map to an egress flow point

In this section, following the steps will configure the parts of the picture highlighted in red.

| PCP COS Color COS Color PCP Forwarding Domain FD1 Flow Point Flow Point FP1 FP2 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Port 1 |  | Port 2 |  |

Procedure:

- Go to the configuration terminal

  - config

- Enter the following commands

In addition to adding the map to the flow point, the map show be used in the l2-trasform if a re-marking of the packets needed.

fps fp FPT logical-port 1 fd-name FD1

stats-collection on

frame-to-cos-map FTC1 egress-l2-transform EL2T1 vlan-stack 1

stamp-pcp map exit

- Go back out of configuration terminal

# exit

- Check if the COS to frame map is associated with the flow point:

  - show flow-points flow-point FPT

You should see something like the following:

+	Flow Point	+

| KEY	| VALUE |

+	+	+

| Name	| FPT	|

| Forwarding Domain Name | FD1	|

| Logical Port	| 1	|

| MTU Size	| 2000 |

| Egress L2 Transform	|	|

|	Egress Name	| EL2T1 |

|	Egress Vlan Stack	|	|

|	Tag	| 1	|

|	Stamp Pcp	| map	|

| Frame To CoS Map	| FTC1 |

+	+	+

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

###### Test payload calculation mode in ingress metering with traffic

Objective:

Objective is to verify ingress metering bandwidth calculation mode application

Procedure:

- Follow the steps 3.3.1 and 3.3.3 to create a meter-profile and attaching it to an ingress flow point

- Run Ixia Traffic and confirm seeing the eir rate

- Change bandwidth calculation mode setting using step 3.3.2

- Confirm a slight increase in the received traffic

- Change back the bandwidth calculation mode using the following commands

bw-calculation-mode iqos-bw-calculation-mode transport bw-calculation-mode eqos-bw-calculation-mode transport

- Confirm a slight decrease in the received traffic

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

Ingress Metering - QoS Flow and QoS meter profile

Objective:

Objective is to create QoS flow and QoS meter profile. Parent interface for QoS flow is on flow point only, logical port as parent interface is NOT supported.

The classifiers that can be used to create QoS flow are:

51xx : single vlan, double vlan, untagged, internal cos, or any

39xx : single vlan, untagged, single vlan + PCP, IPv4 based DSCP, Ipv6 based DSCP, DSCP+Mask, or any

Procedure:

- Create ingress meter profile (~2G) and ensure that the profile can be attached to QoS flow:

  - meter-profiles meter-profile mp2G-qos cir 2000000 cbs 256 eir 10000 ebs 128 color-aware false coupling-flag false shared-meter-instance false valid-interface-type qos-flow

  - meter-profiles meter-profile mp2G-qos2 cir 2000000 cbs 256 eir 10000 ebs 128 color-aware false coupling-flag false shared-meter-instance false valid-interface-type qos-flow

- Create ingress meter profiles and ensure that the profile can be attached to a parent flow point.

  - meter-profiles meter-profile mp7G cir 6000000 cbs 256 eir 500000 ebs 128 color-aware false coupling-flag false shared-meter-instance true valid-interface-type flow-point

  - meter-profiles meter-profile mp4G-fp cir 4000000 cbs 256 eir 10000 ebs 128 color-aware false coupling-flag false shared-meter-instance true valid-interface-type flow-point

  - meter-profiles meter-profile mp2G-fp cir 2000000 cbs 256 eir 10000 ebs 128 color-aware false coupling-flag false shared-meter-instance true valid-interface-type flow-point

- Validate the meter profiles:

5170-0010-R1> show meter-profiles

+	METER PROFILE	+

| Name	| CIR Kbps | EIR Kbps |

+	+	+	+

| mp2G-fp	| 2000000 | 10000	|

| mp2G-qos | 2000000 | 10000	|

| mp2G-qos2 | 2000000 | 10000	|

| mp4G-fp	| 4000000 | 10000	|

| mp6G-qos | 6000000 | 10000	|

| mp7G	| 6000000 | 500000	|

| mp600M	| 200000	| 400000	|

+	+	+	+

- Create the classifier to use for QoS flow. classifiers classifier internalcos8 filter-entry internal-cos internal-cos 8 exit

classifiers classifier anycos filter-entry any

exit

- Create QoS flow that uses the “mp2G-qos” and “mp2G-qos2” meter profiles.

##create QoS flow that classifies on internal Cos of 8 or 16. qos-flows qos-flow qf1-cos8-cos16 meter-profile mp2G-qos

parent-fp CustomerQos classifier-list internalcos8 classifier-list internalcos16 classifier-list-precedence 10 stats-collection on

exit exit

##create QoS flow that is a catch all QoS flow.

qos-flows qos-flow qos-catchall

meter-profile mp2G-qos2 parent-fp CustomerQos classifier-list anycos classifier-list-precedence 11 stats-collection on

exit exit

- Validate the QoS flows.

5170-0010-R1> show qos-flows

+

QOS FLOWS	+

| Name	| Meter Profile | Parent Logical Port | Parent Flow Point | Stats Collection |

+	+	+	+	+	+

| qf1-cos8-cos16 | mp2G-qos	| -	| CustomerQos	| on	|

| qos-catchall	| mp2G-qos2	| -	| CustomerQos	| on	|

+	+	+

+

+

+	+	+ QOS FLOWS STATS

| Name	| Accepted Bytes | Accepted Frames | Yellow Bytes | Yellow Frames | Dropped Bytes | Dropped Frames |

+	+	+

+

+	+	+	+

| qf1-cos8-cos16 | 0	| 0	| 0	| 0	| 0	| 0

|

| qos-catchall	| 0	| 0	| 0	| 0	| 0	| 0

|

+	+	+

+

+	+	+	+

- Next step is to setup meter profiles on parent flow point, please see next testcase.

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

###### Create a queue group instance

Objective:

Objective is to verify creation of a queue group instance

In this section, following the steps will configure the parts of the picture highlighted in red.

Queue 1

Scheduler

Schedule

…

Queue n

Forwarding

Domain FD1

Flow Point

FP1

Flow Point

FP2

Port 1

Port 2

Procedure:

- Go to the configuration terminal

  - config

- Enter the following command

egress-qos queue-group QG1 instance-id 1

parent-scheduler S1 queue-group-profile QGP1 exit

exit

- Go back out of configuration terminal

# exit

- Check if the queue group instance is created:

  - show queue-group-instance

You should see something like the following:

+	Queue Group Instance	+

| Name | Parent Scheduler | Queue Group Profile |

+	+	+	+

| QG1 | S1	|	|

+	+	+	+

Then check:

show queue-group-instances queue-group-instance QG1

You should see something like the following:

+-- Queue Group Instance --+

| KEY	| VALUE |

+	+	+

| Name	| QG1	|

| Parent Scheduler | S1	|

| Instance Id	| 1	|

+	+	+

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

Configure Low Latency Optimization on Egress Queue 5

Low latency optimization is supported on 513x, 5164, 5166, 5168 and 811x platforms only. Low latency optimization, when configured, optimizes the latency performance of an egress queue. Low

latency optimization is configured on a congestion avoidance profile that is then assigned to an egress queue.

Objective:

The objective of this test is to configure low latency optimization on queue 5.

Procedure:

- Configure a low latency optimized congestion avoidance profile:

egress-qos

congestion-avoidance-profile cap_1

type wred config-form custom drop-rate-exponent 1 max-queue-depth low- latency-optimized

curve 1 lower-threshold 100 upper-threshold 100 max-drop-probability

100

curve 2 lower-threshold 50 upper-threshold 100 max-drop-probability 100

- Assign the congestion avoidance profile to queue 5:

egress-qos

queue-group-profile qgp5 cos-queue-map cqm

queue 5

congestion-avoidance-profile cap_1

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

###### Create a FlexE Channel

Objective:

The objective of this test is to create a 25G FlexE channel.

Topology:

Procedure:

Configure a 25 FlexE channel.

      - Create a FlexE channel by assigning 5G calendar slots equal to the bandwidth required. In the following example we are creating a 25G FlexE channel.

flexe-channels flexe-channel channel-1

channel-number 512 group-name group1

channel-mapping L1-mapped

| calendar-A-slots-list | 1 | 1 |
| --- | --- | --- |
| exit |  |  |
| calendar-A-slots-list | 1 | 2 |
| exit |  |  |
| calendar-A-slots-list | 1 | 3 |
| exit |  |  |
| calendar-A-slots-list | 1 | 4 |
| exit |  |  |
| calendar-A-slots-list | 1 | 5 |
| exit |  |  |
| exit |  |  |
| exit Expected Results: |  |  |

A 25 GE FlexE channel should have been created.

5164> show flexe-groups flexe-group group1

+	FLEXE GROUPS	+

| KEY	| VALUE	|

+	+	+

| FlexE Group	| group1	|

| Admin State	| enabled	|

| Group No.	| 1	|

| Calendar Slot Granularity | slot-5G	|

| Phy Type	| flexe-phy-100GBASE-R |

| Calendar	| calendar-A	|

| FlexE Phys	|	|

|	Phy No. In Grp	| 1	|

|	Local Interface	| flexe-port1	|

|	|	|

|	Phy No. In Grp	| 2	|

|	Local Interface	| flexe-port2	|

+	+	+

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

###### WebGUI – Create IS-IS Instance

Objective:

Validate the creation of IS-IS instance using WebGUI.

Procedure:

- On CLI, display the IS-IS summary. There should be no IS-IS instance configured.

show isis summary

5162-004> show isis summary

+ ISIS SUMMARY +

| Name | Value |

+	+	+

|	|	|

+	+	+

- On WebGUI, on navigation pane on left-hand side, click on L3/IS-IS.

- Click on Configure IS-IS on top right hand corner.

- Then click on Advanced Settings. Fill in the necessary information and click on Save.

- In the same IS-IS Advanced Settings, cycle through the tabs and filling any necessary information. Leave blank if default values are appropriate. In this testcase, Interface tab is filled out to enable IS-IS on an IP interface if5. Click on Save when done.

  - In drop down menu in IS-IS, select IS-IS Neighbors to see adjacencies.

  - Validate on CLI as well.

| show | isis | neighbors |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5162-004> + | show | isis neigh ISIS NEIGHBOR STATE |  |  |  |  | + |
| ¦ Neighbor ¦ |  | ¦ ¦ ¦ | ¦ | Hold | ¦ | ¦ | ¦ |

|	Type	| System ID | Interface |	SNPA	| State | Time (s) | Type | Protocol |

+	+	+	+	+	+	+	+	+

|	P2P	| 3926-036 |	if5 | 0023.8afa.6105 |	Up |	29 | L1 | IS-IS	|

|	P2P	| 5166-004 |	if18 | 1892.a455.6704 |	Up |	24 | L1L2 | IS-IS	|

+	+	+	+	+	+	+	+	+

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

###### WebGUI – Enable LDP

Objective:

Validate the enabling of LDP as MPLS transport tunnel using WebGUI.

Procedure:

- From CLI, check for any existing provisioning of LDP.

Show ldp route Show ldp lsp

Show ldp sessions Show ldp adjacencies

5162-004> show ldp route

+	LDP ROUTES STATE	+

| Prefix | Next Hop | IF Name | Path Type |

+	+	+	+	+

|	|	|	|	|

+	+	+	+	+

5162-004> show ldp sessions

+	LDP SESSION STATE	+

| Peer Address | Interface | Role | State | KeepAlive | Targeted LDP |

+	+	+	+	+	+	+

|	|	|	|	|	|	|

+	+	+	+	+	+	+

5162-004> show ldp lsp

+

LDP LSP STATE	+

| FEC	| Nexthop	| State | Label | Req ID | Attr |

+	+	+	+	+	+	+

|	|	|	|	|	|	|

+	+	+	+	+	+	+

5162-004> show ldp adjacencies

+	LDP ADJACENCY STATE	+

|	|	| Hold	|	|	|

| IP Addr | Interface | Time (s) | LDP Identifier | Type |

+	+	+	+	+	+

|	|	|	|	|	|

+	+	+	+	+	+

- On WebGUI, on navigation pane on left hand side, select LDP.

- On WebGUI, select Configure to configure an LDP instance. Click Save.

- After LDP instance provisioning, attach IP interfaces to the LDP instance. Click Save.

- Validate that the interfaces have been created on WebGUI.

- Enable MPLS on the LDP interfaces by clicking on MPLS in navigation pane on the WebGUI. Then click on Attach/Detach Interface. Enable label switching on selected interfaces. Click Save.

- Validate the MPLS and LDP sessions on CLI.

Show ldp adjacencies Show ldp sessions Show mpls ftn-table

Show mpls forwarding-table

5162-004> show ldp adjacencies

+	LDP ADJACENCY STATE	+

|	|	| Hold	|	|	|

|	IP Addr	| Interface | Time (s) | LDP Identifier |	Type	|

+	+	+	+	+	+

| 172.16.18.2 |	if18 |	15	| 66.66.66.140:0 | LINK HELLO |

| 172.17.5.2 |	if5 |	15	| 26.26.26.235:0 | LINK HELLO |

+	+	+	+	+	+

5162-004> show ldp sessions

+

LDP SESSION STATE	+

| Peer Address | Interface | Role	| State	| KeepAlive | Targeted LDP |

+	+	+	+	+	+	+

| 66.66.66.140 | if18	| Passive | OPERATIONAL | 30	| No	|

| 26.26.26.235 | if5	| Active | OPERATIONAL | 30	| No	|

+	+	+	+	+	+	+

5162-004> show mpls ftn-table

+

+

MPLS FTN TABLE

|	|			| FTN |		| Oper | Admin |	|	| Label		|	|		|

| Owner | FEC	| ID | Row Status | Status | Status | Action Type	| OpCode |	In | Out | Out Intf | Next Hop		|

+	+	+	+

+	+	+	+

+	+	+	+	+

| LDP | 26.26.26.235/32	| 2 |	active	|	UP	|	UP	| Redirect to LSP | PUSH	|	-	|

3 | if5	| 172.17.5.2	|

| LDP | 66.66.66.140/32	| 1 |	active	|	UP	|	UP	| Redirect to LSP | PUSH	|	-	|

3 | if18	| 172.16.18.2	|

| LDP | 172.17.4.0/30	| 3 |	active	|	UP	|	UP	| Redirect to LSP | PUSH	|	-	|

3 | if5	| 172.17.5.2	|

+	+	+	+

+	+	+	+

+	+	+	+	+

5162-004> show mpls forwarding

+	+

| Codes: > - installed FTN, * - selected FTN, p - stale FTN, B - BGP FTN, K - CLI FTN, |

|	L - LDP FTN, R - RSVP-TE FTN, S - SNMP FTN, I - IGP-Shortcut,	|

|	U - unknown FTN, T - MPLS-TP Map FTN, O - SR-OSPF FTN,	|

|	i - SR-ISIS FTN, k - SR-CLI FTN	|

+	+

+	MPLS FORWARDING TABLE	+

|	|	| FTN |	|	Label	|	|	|

| Code | FEC	| ID	|	LSP-Type |	In | Out | Out Intf | Next Hop	|

+	+	+	+	+	+	+	+	+

| > L | 26.26.26.235/32	|	2	| LSP_DEFAULT |	-	| 3 |	if5	| 172.17.5.2	|

| > L | 66.66.66.140/32	|	1	| LSP_DEFAULT |	-	| 3 |	if18	| 172.16.18.2	|

| > L | 172.17.4.0/30	|	3	| LSP_DEFAULT |	-	| 3 |	if5	| 172.17.5.2	|

+	+	+	+	+	+	+	+	+

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments

###### WebGUI – Create BGP Instance

Objective:

Validate the creation of BGP Instance from WebGUI.

Procedure:

- On CLI, verify that the BGP instance has not been created.

show bgp

show bgp peers

5162-004> show bgp peers

+

| ¦ ¦ Received ¦ Sent |  | ¦ | ¦ | ¦ Up ¦ | ¦ Peer | ¦ Received | ¦ Advertised | ¦ Last | ¦ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ¦ Prefix | ¦ Remote ¦ ¦ Prefix |  | Peer ¦ Time ¦ ¦ |  | ¦ Table | ¦ Pkt | ¦ Pkt | ¦ Reset | ¦ |

+

BGP PEERS

| ¦ | Peer ¦ | AS | ¦ Type ¦ (hh:mm:ss) ¦ | Version ¦ | Count | ¦ Count | ¦ (hh:mm:ss) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ¦ | Count | ¦ | Count ¦ State ¦ |  |  |  |  |
| + | + |  | + + + | + |  | + | + |

+	+	+	+

|	|	|	|	|	|	|	|	|

|	|	|

+	+	+	+	+	+	+	+

+	+	+	+

5162-004> show bgp

+	BGP	+

| Name | Value |

+	+	+

|	|	|

+	+	+

- On WebGUI, on left hand side navigation pane, select L3/BGP.

- Then select Configure BGP.

- Select MP-BGP for configuration. Fill in the necessary information and then click Save. Can choose Advanced Settings for more detailed info.

- Validate that BGP peers have been added.

- Validate that the BGP peering has been established.

- Validate on CLI as well.

Show bgp

Show bgp peer

5162-004> show bgp

+	BGP	+

| Name	| Value	|

+	+	+

| AS	| 484848	|

| Table version | 1	|

| Prefix count | 0	|

| Router ID	| 62.62.62.223 |

+	+	+

5162-004> show bgp peer

+

BGP PEERS

+

|	|		|	| Up		| Peer		| Received | Advertised | Last	| Received | Sent	|		|

|	| Remote | Peer	| Time	| Table	| Pkt	| Pkt

| Reset	| Prefix	| Prefix |	|

| Peer		| AS	| Type		| (hh:mm:ss) | Version | Count	| Count	| (hh:mm:ss) | Count	| Count | State	|

+	+	+

+	+

+	+	+	+

+	+	+

| 26.26.26.235 | 484848 | internal | 00:06:27	| 1	| 123	| 133

| 00:06:32	| 0	| 0	| Established |

| 66.66.66.140 | 484848 | internal | 00:01:25	| 1	| 4	| 4

| -	| 0	| 0	| Established |

+	+	+

+	+

5162-004>

+	+	+	+

+	+	+

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments