###### SAT testing with inner VLAN ID validation

Objective:

Objective is to configure SAT (Service Activation Test) on a doubled tagged customer traffic by validation inner VLAN ID. Inner VLAN ID validation is either Enabled or Disabled depending on the Reflector capability to reflect traffic with inner VLAN ID.

Procedure:

Following steps needs to be applied.

- Configure EVPL service by using double tag configuration on the UNI port

- Generator and Reflector configurations are slightly different than each other. Generator configuration:

  - Configure SAT Entity

  - Configure SAT KPI Profile

  - Configure SAT Test Profile

  - Configure SAT Test Instance

  - Enable global-sat

- Reflector configuration:

  - Configure SAT Entity

  - Configure SAT Test Instance

  - Enable global-sat

- After Generator and Reflector is configured, “sat-global” needs to be enabled on both

nodes

- SAT test can be started from the Generator node if “sat ping” issued in Generator node

gives successful connection with the Reflector node.

EVPL service creation on Generator and Reflector nodes:

Following configurations will be used on both Generator and Reflector nodes.

config

classifiers classifier dtag filter-entry vtag-stack vtags 1 vlan-id 200 classifiers classifier dtag filter-entry vtag-stack vtags 2 vlan-id 400

classifiers classifier vlan_111 filter-entry vtag-stack vtags 1 vlan-id 111

fds fd fd_sat_inner mode vpws fps fp fp_sat_inner_uni

fd-name fd_sat_inner logical-port 10

mtu-size 9216

classifier-list-precedence 1 classifier-list dtag

exit exit

fps fp fp_sat_inner_nni fd-name fd_sat_inner

logical-port 11

mtu-size 9216

classifier-list-precedence 1 classifier-list vlan_111

ingress-l2-transform pop_111 vlan-stack 1

pop-type exit

exit

egress-l2-transform v111 vlan-stack 1

push-tpid tpid-88a8 push-vid 111

exit exit exit exit

SAT Generator node configuration:

SAT Entity configuration:

config

sat entity ent_gen_inner

port 10

role generator

mode evc-out-of-service admin-state enabled exit

exit

SAT KPI Profile configuration:

config

sat kpi-profile kpi_inner pcp 0 throughput 50000

frameloss 100

latency 1

pdv 2

single-outage-max 1

sum-outages-max 2 exit

exit exit

Provisioned throughput number here provides how much of the total bandwidth can be allocated for overheads as in 4 point of decimal and by percent of this number. For the UNI port 10G rate “50000” entry means, up to % 5 of 10G rate can be used for overheads. So customer should be able to use 9.5G rate of traffic.

Provisioned frame loss number here provides how much loss can be tolerated as in 4 point of decimal and b percent of this number. For the UNI port 10G rate “100” entry means, up to % 0.01 loss is accepted.

SAT Test Profile configuration:

config

sat test-profile prf_inner bandwidth 10000

interval tCompletion duration tOnce

frameloss-test true

inner-vid-validation true vid-validation false

dst-mac 54:C3:3E:B2:F8:8A

kpi-profile-name kpi_inner encap-type qinq

frame-size 80 256 512 1280 1518 exit

exit

“dst-mac” in the configuration will be the “shared-mac” MAC address of Reflector which can be

queried via “show system macs.”

It is suggested to use “duration” as “tOnce” and “interval” as “tCompletion.”

SAT Test Instance configuration:

config

sat test-instance ins_gen_inner

flow-point-id fp_sat_inner_uni profile prf_inner

dst-mac 54:c3:3e:b2:f8:8a c-vid 400

s-vid 200

admin-state enabled exit

exit

“dst-mac” in the configuration will be the “shared-mac” MAC address of Reflector which can be queried via “show system macs.”

#  10.181.36.86 - PuTTY

5170_0038> show sat test-instances

+	SAT TEST INSTANCES	+

I Parameter	I Value

+

Test-Instance-Name

Flow-Point-Id

Profile

S-Vid C-Vid

Untagged

Dst-Mac Ad.min-State Id

Current-Interval Total-Intervals

Last-Iteration-Start-Timestamp

Assoc-Entity Oper-State Active-5-Vid Active-C-Vid Active-Ost-Mac Active-Start-Bw Required-Start-SW Active-Frame-Size

Test-Start-Timestamp

Test-End-Timestamp

Session Keys:

Session-Id Slot-Id Lif-Id S-Vid S-Pcp

S-Color C-Vid C-Pcp

C-Color Untagged

Mask

Profile running configuration:

Interval

OU.ration

Sampling-Interval Max-Samples

Max-Searches Bandwidth

Excess-Bandwidth Frameloss-Start-Bw Frame-Size-List Emix-Sequence

Kpi-Profile

Bli-Alloc-Profile Yid-Validation Color-Validation PCP-Validation VLAN-TPID

VLAN-Encapsulation

Pdu-Type Destination MAC Test Configured:

+

+	+

ins_gen_inner

fp_sat_inner_uni prf_inner

200

400

False

54:c3:3e:b2:f8:8a

enabled

l l l

2020-01-28T21:59:432

ent_gen_inner

disabled 200

400

54:c3:3e:b2:f8:8a

10000

10000.00

(80, 256, 512, 1280, 1518] 2020-01-28T21:59:432

2020-01-28T22:0l:202

0

l

33917

200

0

green

400

0

green

0

11111111

tCompletion tOnce

l

10

7

10000

0

maximum-throughput

(80, 256, 512, 1280, 1518]

Not configured 1.'"pi_inner

Not configured True

True

True

0x8100

qinq

ethernet

54:c3:3e:b2:f8:8a

Throughput-Test Frameloss-Test Latency-Test

PDV-Test

+	+

Enable global-sat

After all the steps above completed, global-sat needs to be enabled.

config

sat sat-global admin-state enabled

SAT Reflector node configuration:

SAT Entity configuration:

config

sat entity ent_ref_inner port 10

role reflector

mode evc-out-of-service reflection-level l2-to-l4

assoc-gen-vendor-type other admin-state enabled

exit exit

SAT Test Instance configuration:

config

sat test-instance ins_gen_inner flow-point-id fp_sat_inner_uni admin-state enabled

exit exit

Enable global-sat

After all the steps above completed, global-sat needs to be enabled.

config

sat sat-global admin-state enabled

Running SAT test

Before running the SAT test, first step is to check whether Generator and Reflector node can

communicate with each other. This can be done via “ping” on the Generator node.

sat ping test-instances test-instance ins_gen_inner

After making sure that “ping” is successful SAT can be started with the following command on the

Generator node.

sat start test-instances test-instance ins_gen_inner

Test results can only be queried after the SAT test is over, till then it is possible to query test status.

show sat test-instances status

Test results will be shows with the following command after the test is over.

show sat test-instances statistics

li1' 10.181.36.86 - PuTTY

51 70_0038> show sat test-instances statistics

+

SATTEST INSTANCES STATISTICS & RESULTS	+

| Test duration + | tOnce + | + | I + + | + | + | + | + + |
| --- | --- | --- | --- | --- | --- | --- | --- |
| I Throughput Results I + I I I I I + I Latency Results I + I I I I I + I PDV Results I + I I I I I i | I Frame I Size + I so I 256 I 512 I 1280 I 1518 + I Frame I Size + I BO I 256 I 512 I 1280 I 1518 + I Frame I size + I BO I 256 I 512 I 12so I 1518 i | I PCP I Color I Min I I I I Mbps I + + + + I o I green I 7750.00 I I o I green I 9130.43 I I o I green I 9624. 06 I Io I green I 9846.15 I I o I green I 9869.96 I + + + + I PCP I Color I Min I I I I usec I + + + + I O I green I B I I o I green I a I I o I green I 9 I I o I green I 11 I I O I green I 11 I + + + + I PCP I Color I Avg I I I I usec I + + + + I o I green I o I I o I green I o I I o I green I o I I o I green I o I I O I green I O I i i i i |  | Avg Mbps 7750.00 9130.43 9624. 06 9846.15 9869.96 Avg usec 379 20 379 166 143 | I Max I KPI Result I Iterations I I Mbps I I I + + + + I 7750.00 I pass I 1 I I 9130.43 I pass I 1 I I 9624. 06 I pass I 1 I I 9846.15 I pass I I I 9869.96 I pass I I + + + + I Max I KPI Result I Samples I I usec I I I + + + + I 767 I pass I 10 I I 29 I pass I 10 I I 764 I pass I 10 I I 326 I pass I 10 I I 277 I pass I 10 I + + + + I I KPI Result I Samples I I I I I + + + + I Ipass I 10 I I Ipass I 10 I I I pass I 10 I I I pass I 10 I I I pass I 10 I i i i i |  |  |

I	I	I	I	I	I	I	I	I	I

+- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -+- - - - - - - - -- - - - - - - - - - - - -+- - - -- +- - - - - - -+- - - - - - - - -+- - - - - - - - - - - - - - - - +-- - - - - - - -+- - - - - - - -- - - - +	+

| Test-Instance-Name Associated-Entity-Name | ins_gen_inner ent gen inner | I I |
| --- | --- | --- |
| Associated-Test-Profile-Name | prf-inn;r | I |
| Test-Bandwidth | 10000 | J |
| Active destination MAC | 54:c3:3e:b2:f8:8a | J |
| Test-Packet Encapsulation Type | qinq | I |
| Active vtag-stack | 200: 400 | J |
| Last Test Start timestamp | 2020-01-2BT21:59:43Z | I |
| Test End timestamp | 2020-01-2 8T22: 01: 20Z | I |
| Port Type | regular | I |
| Test interval | tCompletion | I |

Frames sent by Generator

Frames reflected by Reflector

Test Case Results:

Passed: Yes 	No 	Verified by 	Date/Time	 Comments